SA WG2 Meeting #172	S2-2510125

17 - 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	[WT#5 Data framework] WT scope and KI on 6G Data framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses and proposes work task scope and corresponding KI description for WT#5 on 6G Data framework.

# 1.	Discussion

## 1.1 	AIML feature for 5G in SA2 and SA5

The data framework is supported in different WGs in 5G. SA5 supports the Management Data Analytics to provide the capability of processing and analysing data related to network and service events and status in TS 28.104. SA2 also provides the capability of collecting, processing and analysing data for AIML use cases in TS 23.288. The data management is strongly related to the data source of the input and the consumer of output data for different AIML use cases. However, comparing the supported analytics in TS 28.104 and TS 23.288, there are several similar analytics defined in both WGs. e.g., the Network slice load analysis in clause 8.4.2.5 in TS 28.104 and the Slice load level related network data analytics in clause 6.3 in TS 23.288. The E2E latency analysis defined in clause 8.4.2.4 in TS 28.104 and the Redundant Transmission Experience related analytics defined in clause 6.13 in TS 23.288.

Observation 1: There are several similar AIML use cases with similar input data and output data defined in SA2 and SA5. The data management which includes data collection, data storage and data exposure are strongly related to the specific use cases for AIML.

UE data collection for UE side model training is the feature requested by RAN plenary to be studied both in SA2 and SA5. It is still under discussion regarding different solutions to transfer data from UE to 5GC / OAM and then UE side server. It is also in RAN2’s scope to decide what solution(s) will be adopted in 5G.

Observation 2: UE data collection for UE side model training is the feature under discussion in both SA2 and SA5 for 5G.

The dataset / parameter transfer for two-sided model is another example that involved both SA2 and SA5, it is still under discussion in both SA2 and SA5 whether and how to capture in the Rel-20 scope.

Observation 3: The dataset / parameter transfer for two-sided model is also under discussion in both SA2 and SA5 for 5G.

The WT related to data framework is under discussion in SA2 and SA5, SA2 almost get a consensus to start with the use case discussion and almost we all agree that the coordination with SA5 is required to avoid the duplicate work between two WGs.

Proposal 1: It is required to have a check point to coordinate with SA5 after the use case discussion in SA2.

## 1.2	Sensing discussing in 5G and 6G

In 5G-A, the sensing data from the gNB is not per UE, but per nodal level data. This is different from the case for LCS, where the data collected from the gNB is for the UE. In addition, different sensing architecture, authorization procedure, sensing entity discovery are discussing in 5G for sensing service, which are also different from that of the LCS. The sensing data collection depends on the sensing architecture and the format and the content of sensing data depend on RAN’s decision. Therefore, there is no common data collection mechanism for even similar services like Sensing and LCS.

For 6G Sensing study, there is already a WT#4 (S2-2509961), which includes specifically the tasks for data collection and exposure:

a.4) Collection and transport of sensing related data.

a.5) Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

Additionally, the scope of WT#4 needs to align with RAN at the checkpoint at Q1.

NOTE 7: The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

Therefore, it is not reasonable to discuss the data handling for sensing in WT#5 before WT#4 finishes their work . Otherwise, it would cause parallel and potentially conflicting work in two work tasks, as WT#4 may update scope and change architecture design for sensing at a later phase.

Proposal 2: The data handling for sensing data should be discussed in WT#4.

## 1.3	User Consent

In SA3 ToR the following is defined:

" The WG will further accommodate, as far as is practicable, regional regulatory requirements that are related to the processing of personal data and privacy."

Also in LS from SA#109 (SP-251261) the following is defined:

" SA2 and SA5 should coordinate with SA3 for the data security and privacy requirements."

It is not SA2’s responsibility to analyse and determine whether to satisfy the national and regional regulations regarding user consent, SA2 should only study the potential system impacts to satisfy the requirements from SA3.

Proposal 3: SA2 should only study the potential system impacts based on SA3’s requirements.

# 2. 	Proposal

It is proposed to approve the following WT scope in TR23.801-01.

**** First Change ****

# A.X 	Work Task X: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA5 is required. Common functionalities for data handling may be identified between SA2 and SA5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

- Identify high level use cases for 6G assuming the use cases in 5GS as starting point for discussions and potentially new use cases subject to the use of data framework.

NOTE 2: This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3: The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

NOTE 4:	The use cases of WT#5 are required to coordinate with SA5 for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit the use cases before discussing solutions.

- Study required data framework functionalities to support the identified use cases, e.g:

- data discovery and data registration

- data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution) )

- data labelling/metadata handling

- data processing (e.g.data anonymization, data analysis, data generation, etc.)

- data storage and retrieval

- data exposure

The functionalities above may need to consider the quality of data, latency, data volume.

NOTE 5:	 Coordination with SA5 may be required for charging aspects.

NOTE 6:  Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

- Study potential system impacts of user consent, privacy, security and data governance from SA2 architectural and system level perspective for data generated in 6G system based on SA3’s requirement.

NOTE 8:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA3. SA2 should consider the output of SA3 before specifying any system procedure on this topic.

**** End of Changes ****