3GPP TSG-SA WG2 Meeting #172	S2-2510129

13 - 17 November, 2025, Dallas, USA

Source: 	China Mobile

Title: 	Interim conclusion for KI#3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC and KI#4 Location service for IMS emergency call and regulatory services over NB-IoT NTN

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an updated interim conclusion for KI#3 and KI#4 of FS_5GSAT_Ph4_ARC.

1	Discussion

- Following the previous agreements on KI#1 and the latest analysis of emergency and regulatory service support over NB-IoT NTN via GEO satellites, this discussion focuses on Key Issue #3 and Key Issue #4 defined in TR 23.700-19 V1.1.0

- Key Issue #3 – IMS Emergency Call over NB-IoT NTN via GEO Satellite

- For IMS emergency service support over NB-IoT NTN, it was observed that the EPC-based GEO architecture and IMS emergency call procedures defined in TS 23.167 can mostly be reused. The following aspects were highlighted:

- For UE-detectable emergency calls, the standard IMS emergency registration and session setup can be applied directly, with only small adjustments to handle GEO satellite delay and coverage.

- For non-UE-detectable emergency calls, except for home-routed cases, the call can be treated as a normal IMS session while being marked as an emergency call by the network.

- Key Issue #4 – Location Service for IMS Emergency Call and Regulatory Services

- For KI#4, the study confirms that the existing LCS architecture defined in TS 23.271 can be reused to provide UE location information for IMS emergency calls over NB-IoT NTN GEO.The following aspects were highlighted:

- The current LCS architecture defined in TS 23.271 can be applied without new interfaces, using existing EPC and IMS functions for location retrieval and delivery.

- The UE-provided location remains the primary source; when not available or inaccurate, the network may obtain or verify the location through EPC-based or GMLC/LRF-assisted methods.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-19 v1.1.0.

* * * First Change * * *

## 8.3	Interim conclusions for Key Issue #3

Editor's note: Other interim conclusions for Key Issue#3 is FFS.

The following interim conclusions for Key Issue #3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC are made:

-	For UE-detectable emergency calls, existing IMS emergency call procedures in TS 23.167 can be reused.

-	For non-UE-detectable emergency calls, except for the home-routed case, the call is established as a normal IMS session and treated by the network as an emergency session internally.

## 8.4	Interim conclusions for Key Issue #4

Editor's note: Other interim conclusions for Key Issue#4 is FFS.

The following interim conclusions for Key Issue #4 Location service for IMS emergency call and regulatory services over NB-IoT NTN are made:

-	The existing LCS architecture and procedures defined in TS 23.271 can be reused to support the provision of UE location information for IMS emergency calls over NB-IoT NTN. The UE-reported location is primarily used, while the network may obtain or verify the UE’s position through assistance mechanisms to ensure regulatory compliance.

* * * End of Changes * * *