SA WG2 Meeting #172	S2-2510131

17 – 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	CATT

Title:	[WT#1.1] Key issue for Work task #1.1 Support of 6G Control plane signalling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the key issue for work task 1.1 based on the discussion on the pCR (S2-2509819).

1. Justifications

Based on the offline discussion, the work task of WT#1.1 as follows is the version with less comment compared with other versions. Based on the following work task, this pCR proposes to add the key issue for WT#1.1.

Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT3, WT4, WT5, WT6.

* * * First Change (all text new) * * *

5.X	Key Issue #X: Study the support for control signalling for 6G System

The key issue is divided into two parts as follows.

NOTE 0: 	The structuring of the key issue does not imply any restriction on solutions. Same or different solutions can be proposed for the two parts.

Part 1:

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

Part 2:

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT3, WT4, WT5, WT6.

* * * End of Change * * *