3GPP TSG-SA WG2 Meeting #172	S2-2510164

Dallas, US, November 17 - 21, 2025	(revision of S2-250xxxx)

Source:	Sony

Title:	[KI#1] Interim agreement update, UE capabilities

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution addresses the Editor’s note on UE reader capability.

1. Introduction

There is an Editor’s note about UE capabilities.

Editor's note:	Whether UE reader capability is provided to AMF is FFS.

Part of the discussion in SA2#171 was around whether the AMF need to know specific ambient IoT radio capabilities before authorizing the UE to act as a UE Reader.

The current logic in the 5G system is that the AMF gets the UE radio capability information from the NG-RAN (who gets it from the UE) and stores this in the UE context as long the UE is registered. The AMF provides the UE Radio Capability Information to the NG-RAN during the Initial UE context establishment in NG-RAN.

This IE is not used by the AMF, the AMF only stores the IE and provides it to the NG-RAN when UE moves the RRC-Connected (when UE context is established in NG-RAN).

Furthermore, for ProSe UE authorization the subscription information does not include any specific sidelink radio capabilities.

Based on at least these two observations, we can assume that the AMF would not need any specific ambient IoT UE Radio Capabilities to authorize the UE to act as UE Reader.

NG-RAN will, if needed, specify how to handle specific Ambient IoT UE radio capabilities when assigning Ambient IoT radio resources to the UE Reader, without any involvement with the Core network. If the UE Reader is authorized by the core network, but does not support the ambient IoT UE Radio Capabilities required, then the NG-RAN will not use this UE Reader. If the AIOTF selected this UE Reader, the NG-RAN will reject the request to use this UE Reader.

Proposal 1: The AMF will not need to know any specific UE Ambient IoT Radio capabilities to authorize a UE to operate as a UE Reader.

When it comes to the 5GMM Capabilities, it could be discussed whether UE Reader must be part of the 5GMM Capabilities IE or not. To operate as a UE Reader the UE must have a supporting subscription and have the device capabilities. As the USIM can be move to a new device it is preferred that the UE indicates in the initial registration that it has the capability to act as a UE Reader. That will enable the AMF to provide the authorization indication to NG-RAN after checking both the subscription and UE capability.

Proposal 2: The UE includes a UE Reader capability indicator in the 5GMM Capability IE.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change * * * *

### 7.1.1	Agreed Principles for KI#1

'Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2: AIoT System Architecture for Topology 2 in reference point representation

NOTE X:	Whether the interface between AIOTF and UDM is needed will be checked in later phase.

Message and protocol stack

-	Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-3.

Figure 7.1.1-3: Protocol Stack for the RRC option

UE reader authorization and revocation part:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor's note:	Whether and what additional subscription information for the UE Reader is needed is FFS.

-	UE Reader subscription information is available to AMF.

-	The UE includes a UE Reader capability indicator in the 5GMM Capability IE when registering to the network.

NOTE X: RAN WGs might specify that Ambient IoT radio capabilities should be included in the UE Radio Capability Information provided by the UE.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader if the UE indicated UE Reader capability in the 5GMM Capability IE.

Editor's note:	Whether to inform the UE about the authorization information is FFS.

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

Editor's note:	Whether and how UE Reader subscription information is provided to AIOTF is FFS.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

AIOTF Discovery and Selection for UE reader ID:

Editor's note:	How to discover and select AIOTF when AF provides UE reader ID is FFS.

UE reader selection part:

Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader.

-	For AF providing UE reader ID case:

-	If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN.

Editor's note:	How to select UE reader(s) based on the AF provided area information is FFS.

UE reader ID allocation:

Editor's note:	Whether and how to allocate the UE reader ID is FFS..

* * * End of Change * * * *