3GPP TSG-SA WG2 Meeting #172	S2-2510168

Dallas, USA, November 17 – 21, 2025

Source: 	MediaTek Inc.

Title: 	Interim conclusion for KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#2 of FS_5GSAT_Ph4_ARC regarding IMS enhancement for GEO NB-IoT NTN access.

1	Discussion

In TR 23.700-19, around 4 KI#2 solutions directly mentioned pre-configuration or pre-negotiation of media parameters (port, codec etc.) in IMS registration flow to optimize overall call setup time and to achieve KPI from SA1.

SDP body is conventionally used to negotiate media related parameters such as codec, port number during call setup flow. However, there can be plenty information carried in SDP therefore increasing the packet size of SIP message. Considering the limitation of NB-IoT NTN access, more information in SIP message during call setup leads to longer call setup time. Completing the required parameter configuration in IMS registration flow can mitigate this problem.

On the other hand, in SA2#169 and SA2#170, many solutions also mentioned to

- activate dedicated bearer for media stream earlier that is, before making a call

- reduce contents of a SIP message and rely on P-CSCF to restore the required fields.

However, without knowing the port or IP address, extra message exchange or update flow is required to update the pre-established dedicated bearer after SDP negotiation in call. And how to have P-CSCF complete the omitted information during call setup is also a problem to solve. If the required media parameters can be pre-configured in IMS registration flow, then dedicated bearer can be activated according to the pre-configured parameters and the P-CSCF can also be configured with the context to completing omitted information in-call in such flow.

According to the two direction of SIP optimization framework, that is, text-based SIP with content reduction or binary-encoded SIP, pre-configuring media parameters can be integrated based on these two frameworks to reduce CST and simplify call setup flow.

In discussion paper S2-2510167, a detailed discussion over media parameter pre-configuration is provided.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change* * *

8	conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

8. X	Interim Conclusions for KI#2

Editor's note 1:	Other interim conclusions for Key Issue#2 is FFS.

The following interim conclusions for Key Issue #2 IMS enhancement for GEO NB-IoT NTN access are made:

-	Parameters such as codec or port numbers for IMS voice call over NB-IoT NTN is pre-configured during IMS registration flow.

# * * * * End of change * * * *