3GPP TSG SA2 Meeting #172 	S2-2510217

Dallas, USA, 17 – 21 November, 2025	(revision of xx-yyxxxx)

# Source:	Ericsson

# Title:	New WID on Minimized the Number of Policy Associations Phase 2

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Minimized the Number of Policy Associations Phase 2

Acronym:	TEI20_MINPA_Ph2

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| TEI19_MINPA | Minimize the Number of Policy Associations |  |

# 3	Justification

The functional description on the handling of the UE Policy Association in clause 6.1.2.2.5 in TS 23.503 specifies in NOTE 3: “Before setting the indicator to "disabled", the operator needs to ensure that there are no UE Policies stored for any of these services in the UE, by revoking the authorization for the service specific information, for Service Type "AF guidance for URSP", for all of the AFs with active authorizations, as defined in clause 4.15.6.7a of TS 23.502 [3] and by triggering the PCF to remove the UE Policies in the UE if any or both.”

There may be cases where the operator may decide not to trigger removal of UE Policies from the PCF prior to setting the indication to “disabled”, instead a solution that may not require the operator to check that the UE Policies are removed is prefered.

# 4	Objective

To enhance the handling of the UE Policy Association to ensure that even if the UE Policy Association Indicator in the UDM subscription is set to “disabled”, the PCF is informed that the UE Policies needs to be removed because of the change of the subscription values.

The proposed solution includes the following impacts:

- An indication that subscription values for the UE Policy Association Indicator changed from “enabled” to “disabled” is added to UDM.

- If present, it indicates to the AMF that the PCF shall be informed, via PCRTs, when the UE Policy Association is established or the UE Policy Association shall be established to request the PCF to remove the UE policies.

- If present, it indicates to the PCF that the UE policies shall be removed from the UE and the UE Policy Association shall be terminated.

- When the AMF receives a request for termination of the UE Policy association from the PCF and the notification that the UE policies are removed in included, the AMF updates the UE Policy Association subscription indicator in the UDM.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | Updates to clause 4.16 UE Policy Association Establishment to describe a new condition for the AMF to decide if the UE Policy Association is to be established. Updates to clause 4.16 UE Policy Association Termination to describe that the PCF removes the UE Policies from the UE based on AMF requests. Updates to clause 5.2.3.3.1, the subscription data in UDM to include a new indicator that is set when the subscription data for UE Policy Association indicator changed and the UE Policies shall be removed. Updates to clause 5.2.5.6 the Npcf_UEPolicyControl service operators to include the request from the AMF to remove UE Policies and for the PCF to confirm the removal of UE Policies Updates to clause 5.2.3.3.6 the .Nudm_SDM_Info service operation to include the confirmation to UDM that UE policies are removed. | TSG#112 |  |
| 23.503 | Update to the clause 6.1.2.2.5 about the Handling of the UE Policy Association to indicate the conditions for the AMF to establish the UE Policy Association if the UE policy association indicator is set to disabled. | TSG#112 |  |

This TEI20_WID requires 0.5 TU

# 6	Work item Rapporteur(s)

Pancorbo, Belen, Ericsson

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Not applicable.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Ericsson

Verizon