3GPP TSG-SA WG2 Meeting #172	S2-2510232

Dallas, USA, 17th – 21st Nov, 2025

Source:	Ericsson

Title:	KI#3 interim agreements, Energy Saving state scheduling

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Discusses Energy Saving state scheduling and concludes that it is not needed.

# 1. Discussion

3GPP TS 28.310 defines energySaving and notEnergySaving states that are intended to describe the operational modes of network functions (NFs) or resources with respect to their energy consumption. They also describe the process of moving between the states as ES activation/deactivation:

From TS 28.310:

energySaving state: state in which a cell or network function is powered-down for energy saving purposes.

Note 1: 	In energySaving state, the cell or network function is still controllable.

Note 2: 	Void.

notEnergySaving state: state when no energy saving is in progress.

Note 3:	Void.

ES activation: procedure to power down a cell or network function for energy saving purposes.

Note 4: 	As a result, the cell or network function goes into energySaving state.

ES deactivation: procedure to power up a cell or network function.

Note 5: 	As a result, the cell or network function goes into notEnergySaving state.

These SA5 states and transition procedures have not been implemented for CN NFs and the relation between the SA5 defined states in TS 28.310 and the CT defined NF Status in TS 29.510, see below, is unclear.

What is clear, is that an NF in energySaving state will continue to consume (idle) energy since it still need to keep (some of it’s) compute, memory and networking resources to remain “controllable”. Ironically this will also mean that during ES activation (the period while it’s reducing the traffic it carries) the energy per bit will increase and when it reaches the energySaving “standby” state it will consume idle energy without carrying any traffic.

To really save energy for the operator, the NF’s that does not carry traffic need to be shut down and their compute, memory and networking resources should be released.

There seems to be no need to introduce support for the SA5 defined states for CN NFs as the existing NF status management according to 29.510 can handle the cases of taking and NF in and out of service and shutting it down.

What could be investigated as an alternative to scheduling would be e.g. to extend the shutdown timer concept so that it applies to other NF states beyond UNDISCOVERABLE state. This would allow for longer advance warning to NF consumers that an NF will be taken out of service. This alternative aims to improve energy efficiency by better aligning NF state management with the new energy-saving mechanisms, reducing wasted energy consumption during low or no traffic periods.

Based on the discussion above it’s proposed to remove EnergySaving/notEnergySaving state scheduling from the items for further consideration.

# 2. Text Proposal

It is proposed to agree the following to 3GPP TR 23.700-67

First change

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow.

-	In case of multiple configured energy policies in network, the Energy Priority is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

# * * * * End of changes * * * *