3GPP TSG-WG SA2 Meeting #172	S2-2510234

November 17 – 21, 2025, Dallas, TX, USA

Source:	Ericsson

Title:	KI#1, Sol#28 Update: Clarify single IP/UDP flow support between PGW and IMS-AGW

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: This contribution proposes an update to solution#28 in TR 23.700-19, v1.1.0, to clarify single IP/UDP flow between PGW and IMS-AGW.

# Discussion

In solution#28, the PGW converts UL non-IP data into IP format and forwards it to the IMS-AGW. Since the PGW transparently passes the non-IP data, including the new RTP header, only a single IP/UDP header can be added, i.e. PGW does not differentiate the non-IP data. This IP/UDP header is determined by the IP flow description provided by the PCRF when the dedicated bearer for IMS voice is activated.

So, it is proposed to clarify that a single IP/UDP flow is supported in solution #28, as below.

* * * Start of change * * * *

## 6.28	Solution #28: Non-IP/UDP transport for IMS voice media using IP-type PDN Connection

### 6.28.0	High-level solution Principles

In the case of strictly limited bandwidth (e.g. 1kbps) in GEO satellite link, transport of the voice media traffic without IP/UDP header may reduce the protocol overhead and improve the voice quality.

NOTE 1:	The terminology 'non-IP data' used in this paper only refers to packet without IP/UDP header.

NOTE 2:	When voice media is sent without UDP/IP, the RTP header will not be subject to ROHC but will be sent as e2e payload between IMS AGW and UE. This causes overhead that may in some cases be larger than when ROHC is applied for RTP/UDP/IP headers. It is assumed that a new RTP header design by SA WG4 can mitigate the potential protocol overhead issues when applying non-UDP/IP transport of voice media.

Editor's note:	It is FFS and up to SA WG4 whether and how to reduce the overhead in the RTP header.

This solution addresses Key Issue #1 and covers the aspects how to integrate the new function of non-IP transport with existing VoLTE solution. Other aspects are to be addressed by other solutions.

The solution principle is similar to the principle of Solution #3:

-	One IP type PDN connection established for IMS, using UP.

-	SIP messages are transported using default EPS bearer.

-	Voice media is transported by non-IP data between UE and PGW, using a dedicated EPS bearer of the IP type PDN connection.

-	Voice media is transported by IP data as for VoLTE between PGW and IMS-AGW. This solution supports only single IP/UDP flow per UE between PGW and IMS-AGW.

Figure 6.28.0-1: EPS bearers for IMS over NB-IoT (GEO)

### 6.28.1	Description

In this proposal, PDN Connection to IMS APN, with IP type and two bearers, is used for SIP signalling and voice media. The SIP signalling is delivered via default bearer with IP data and voice media traffic is delivered via dedicated bearer using non-IP data between UE and PGW.

### 6.28.2	Procedures

The UE establishes IP type PDN Connection for IMS APN and indicates its support of transporting voice media traffic without IP/UDP header over a dedicated bearer in the PDN connectivity request message, in PCO IE. MME and SGW forwards the UE's capability to PGW via Create Session Request message.

NOTE:	It is assumed that PGW selection is done based on APN as described in current TS 23.401 [5],

When an IMS voice call is initiated, the PGW receives the RAR (Policy and Charging Rules Provision) from PCRF as a trigger to establish a dedicated bearer. The PGW initiates the dedicated bearer activation procedure and informs the UE via SGW/MME that the dedicated bearer is for transporting voice media traffic without IP/UDP header. The IMS voice traffic is handled as follows:

-	For UL voice traffic, the PGW adds the IP/UDP header based on the IP flow description that it has received in PCC rule from PCRF. The IP flow description includes a single IP/UDP flow. There are no impacts to the PCC rule due to this.

-	For DL voice traffic, the PGW matches the IP flow using downlink packet filter and associates the traffic with the dedicated bearer but removes the IP/UDP header before transmitting the packet onto the GTP tunnel.

-	The new RTP header is carried transparently via the PGW in both UL and DL.

-	For UL voice traffic, the UE is provided by PGW with uplink packet filter in TFT and an associated indicator for the dedicated bearer. This allows the UE to associate the non-IP data with the dedicated bearer or convert IP to non-IP before transmitting to the dedicated bearer, depending on the UE internal logic.

-	For DL voice traffic, the UE is provided by PGW with downlink packet filter in TFT and an associated indicator for the dedicated bearer. The UE receives DL packet without IP/UDP header from the dedicated EPS bearer and then the non-IP data may be forwarded to upper layer application directly or converted to IP data first, depending on the UE internal logic.

-	MME indicates to eNB that the bearer is of nonIP type (using existing S1AP bearer type parameter)

Figure 6.28.2-1: Voice media transport without IP/UDP header

### 6.28.3	Impacts on Services, Entities and Interfaces

UE:

-	Indicates in PDN connectivity request its capability of transporting voice media traffic without IP/UDP header.

-	Sending and receiving IMS voice media as non-IP data, per indication from the network.

PGW:

-	PGW-C/PGW-U activate dedicated bearer for transporting voice media traffic without IP/UDP header.

-	PGW-C provides indication associated with the dedicated bearer to handle non-IP data over a dedicated bearer.

-	PGW-C instructs PGW-U how to add/remove IP/UDP header for dedicated bearer of IP type PDN.

-	PGW-U performs IP/UDP header removal/addition when receiving and sending data from/to SGi for dedicated bearer of IP type PDN Connection.

MME/SGW:

-	Support NB-IoT with dedicated bearer.

-	Support dedicated bearer of an IP type PDN connection for transporting non-IP data.

RAN:

-	support NB-IoT with dedicated bearer.

-	support radio bearer which is used for IMS voice media to transport non-IP data.

* * * End of change * * * *