SA WG2 Meeting #172	S2-2510307

17 - 21, November, 2025, Dallas, USA

Source:	T-Mobile USA

Title:	[WT#3.1] AI for 6G architecture

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This pCR proposes some changes to the AI for Network Working task scope and key issue based on S2-2509836.

# 1	Discussion

This paper provides some comments against the current summary of proposals as of SA#171. The focus is on WT#3.1 AI for Networking in the area of AI Agent to Agent communication and the inclusion of intent in this communication.

The advancement of AI technology presents promising opportunities for enhanced performance and improved automation. Nonetheless, from the perspective of network operators, the introduction of variable interactions among network components introduces several concerns. Testability is an essential prerequisite for network deployment; operators must have the capability to execute comprehensive test cases prior to implementation, with consistent and accurate results each time. System outcomes must remain predictable and free from variability.

Within a single Network Function (NF), vendors are at liberty to implement AI technologies as they see fit, potentially incorporating numerous AI agents as part of their internal design. However, external interfaces to any NF must be defined with clarity and precision; reliance on natural language or abstract methodologies is not acceptable. While procedural message flows may exhibit some degree of variability—permitted under 5G standards—the system as a whole must achieve its objectives deterministically. This ensures that test cases can reliably assess system correctness, with behaviors that are both reproducible and consistent.

Furthermore, external communication with an NF may occur through messages, APIs, or similar mechanisms, provided all encodings and parameters are unambiguous and precisely defined, leaving no room for misinterpretation. It is imperative to maintain strict adherence to NF boundaries, prohibiting one NF from remotely initiating internal operations within another. Communication between NFs must occur exclusively through standardized external interfaces.

# 2	Proposal

It is proposed to include the below changes into WT#3.1 scope and key issues.

First Changes

# Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs optionally include intent, determine how to define the external interfaces of all NFs that process intents precisely and with no ambiguity and without relying on natural language or abstract methodologies, so that there is no room for misinterpretation of the request.

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.2	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any, including how to determine the interaction of UEs, AFs, and NFs to behave in a deterministic way that supports running network test cases prior to deployment with consistent and accurate results every time.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.3	determine how to support requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations).

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.4	design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not AI technologies can interwork with home network using or not AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

NOTE 7: It is assumed that operators can decide whether to deploy AI capabilities in their network.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

Editor’s Note: Similar text as above is assumed for the Key Issue.

End of Changes