3GPP TSG SA2 Meeting #172	S2-2510313

# Dallas, USA, November 17-21, 2025

# Source:	Oracle, Verizon

# Title:	New WID on Deferred PRA for All Policies in the 5GC

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Deferred PRA for All Policies (SM/AM/UE) in the 5GC

Acronym:	TEI20_DPAP

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The access and mobility related policy control encompasses the management of service area restrictions, the management of the RFSP Index, the management of the UE-AMBR, the management of the UE Slice-MBR, the slice replacement management and the management of SMF selection.

UE policy encompasses the management of ANDSP, URSP, V2XP, ProSeP, RSLPP, A2XP (see 23.503 clause 6.1.2.2). In the context of this document, UE policies refer to URSP only.

The session management related policy control provides the functions for policy and charging control as well as event reporting for service data flows.

PRA stands for Presence Reporting Area. This area can be composed of Tracking Areas (TAs), eNodeBs, Cell Global Identity (CGI), or other geographical identifiers.

PCF subscribes to reporting change of UE presence in a PRA. Based on the UE presence (or non presence) in the PRA the PCF may take some AM/UE/SM Policy actions.

Such action may include for example:

UE policy - push to the UE a new or updated URSP rule that include a slice/DNN to establish a new PDU session on.

AM policy – Adjust service area restrictions, adjust RFSP, change UE AMBR and so on.

SM policy – Change session AMBR, adjust 5QI, change charging keys and so on.

Some scenarios however require the use of PRA reporting (and the derived policy actions) only during specific intervals. For instance, PRA reporting will be relevant only during an event (e.g a concert) that may take place at a certain area in a specific time for a known duration. Such a selective/automatic/limited PRA reporting can save a lot of unnecessary signalling between the AMF and PCF or/and between the SMF and PCF or/and between the AMF and SMF.

The following scenarios have been identified:

Scenario 1: Concert – A concert will take place at a location X at time T for 2 hours. During that time entitled subscribers that attend the concert will have their session AMBR boosted up and/or their 5QI may get adjusted. PCF for the PDU session may ahead of time instruct the SMF to report it about deferred PRA events (via the existing PRA PCRT (PRA=X, deferred time=T, Duration=2 hours)), e.g subscriber enters the area or leaves it during the time of the concert. Once the concert starts, the PCF will receive PRA reporting. It will then be able to dynamically respond to SMF with proper PCC or/and session rule(s). When the concert ends, the SMF automatically stop sending the PCF PRA reports.

Scenario 2: Similar to scenario #1. However, the PRA reporting is done by the AMF and the PCF is the PCF for a UE. Based on the PRA reports, the PCF can dynamically adjust AM or UE policy, e.g adjust UE AMBR or/and provision URSP rule with DNN/S-NSSAI that allows the subscriber to establish on-the-fly a proper PDU session (or either tear it down if the URSP rule is being removed or updated).

# 4	Objective

- WT-1: The objective is to enhance the PRA reporting mechanism such that it can work (also) in a deferred mode (ie it will have an activation time and/or de-activation time). This enhancement will be applicable to all – SM, AM and UE policy associations.

The scope of this Work Item in the context of SM and UE policies applies to either non-roaming or roaming. The scope of this Work Item in the context of AM policies applies to non-roaming only. Impacted NFs are AMF, SMF, PCF for a UE and PCF for the PDU session. No impact on UE or RAN.

## TU estimates and dependencies

| Work Task ID | TU Estimate | TU Estimate | RAN Dependency | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
|  | (Study) | (Normative) | (Yes/No/Maybe) |  |
| WT-1 |  | 0.5 | No |  |

The expected time for completing this work is 1 TUs. 0.25 time slots in one SA2 meeting to define the functional description and 0.25 time slots in next SA2 meeting to define the procedures and services.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 | Define the impacts on PCF for a UE and AMF. 6.1.2.5, 6.5 - Update the PRA PCRT for Access and mobility policy control and for UE policy control such that they will include an optional activation/deactivation time. 6.6.7 - Update the PRA PCRT for UE policy control such that they will include an optional activation/deactivation time. Define the impacts on PCF for the PDU session and SMF. 6.1.3.5, 6.4 - Update the PRA PCRT for Session Management policy control such that they will include an optional activation/deactivation time. | TSG#110 (Dec. 2025) |  |
| TS 23.501 | Define the impacts on AMF and SMF. 5.6.11 - Update the PRA such that it will include an optional activation/deactivation time. | TSG#110 (Dec. 2025) |  |

# 6	Work item Rapporteur(s)

Baniel, Uri	uri dot baniel at oracle dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Oracle

Verizon UK Ltd