SA WG2 Meeting #172	S2-2510340

17 - 21 Nov 2025, Dallas, TX, USA

Source:	TCL

Title:	[WT#1.1] Proposal for WT and KI of Support of 6G Control signalling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope and Key Issue update of Work task 1.1.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

2. Discussion

In former e-mail discussion it is agreed to give the definition for “NAS” to keep the further study concentrated. There are opinions to use the definition from 24.501 or 21.905. In 24.501, it claims NAS as “the highest stratum of the control plane” between UE and AMF; while in 21.905 it claims NAS as “Protocols between UE and the core network that are not terminated in the UTRAN”. With the development of 3GPP network architecture and introduction of protocol such as LCS-CP, the wording “highest stratum” is not precise. While the definition of 21.905 may include too much items outside the control plane which are not proposed to be studied in this work item.

Another observation is that companies don’t want to limit the solution to carry the traffic for operator services. While considering using UP or new service or NAS transport carrying new service data, we shall not ignore the possibility to use NAS transport to carry control signalling for new services. So, it is not suggested to limit NAS into only NAS-MM and/or NAS-SM. All the containers transferred by NAS carrying control signalling shall be considered as part of NAS.

Here we propose to combine the definition from 24.501 and 21.905 and introduce the following change:

Proposal 1: Non-Access Stratum: Protocols form the control plane between the UE and the 6G CN.

In addition, for Key Issue 1.1.2, we want to highlight that the mobility management and service continuity is essential for all operator service. No matter what solution is going to be raised, these are two baseline requirements to be fulfilled.

Proposal 2: Highlight service continuity and mobility management for all operator services.

**** First Change ****

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Access Stratum(NAS): Protocols form the control plane between the UE and the 6G CN.

Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

Key Issue #1.1 Study the support for control signalling for 6G System

Key Issue #1.1.1

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1: 	It is assumed that this key issue covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

Key Issue #1.1.2

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services. The service continuity and mobility management for operator services shall be ensured.

NOTE 3: This KI1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. KI1.1 bullet 2a can have dependency on other key issues defined for e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

NOTE 4:	Interdependency of NFs within 6G CN to support the 6G System procedure is to be minimized, considering also the efficiency of e2e communication.

NOTE 5: 	6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signaling interaction between RAN-CN need coordination and alignment with RAN3.

NOTE 6: Interworking and migration aspects, where applicable, are covered by key issues for WT#2. Impact to IWK due to features introduced by this key issue will be covered by this key issue.

**** End of Changes ****