3GPP TSG-SA WG2 Meeting #172	S2-2510380

17- 21 November, 2025, Dallas, US(

Source: 	ZTE, CMCC

Title: 	Interim conclusion for KI#2 of FS_5GSAT_Ph4_ARC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#2 of FS_5GSAT_Ph4_ARC regarding the evaluation of different IMS optimization options.

1	Discussion

In TR 23.700-19, 10 solutions are identified to enhance IMS for GEO NB-IoT NTN access, covering several different options regarding the PDN connection and IMS signaling optimization.

For IMS signalling, there are two main dimensions of classification: IMS signalling over IP and IMS signalling over non-IP, text-based simplification and binary-based simplification. These two dimensions are orthogonal and independent of other WG.

For the IMS signalling over IP and IMS signalling over non-IP:

- The size of the IP/UDP header is 28–48 bytes. Under a 2 kbps transmission rate, a single signaling message only incurs an additional delay of 0.112s or 0.192s. With simplified procedures, using non-IP packet would at most save 0.224s to 0.384s, which is negligible. However, it introduces significant system complexity.

- According to TS 23.401, NIDD is a data transmission method with PDN Type "Non-IP" and UE is agnostic of the IP address, making it unsuitable for IMS over GEO. Additionally, only UDP/IP encapsulation/decapsulation at the P-GW is standardized, which does not support IP/UDP header insertion and removal at the UE, thus necessitating additional modifications.

- For IP-type PDN connections, the removal or insertion of specific fields should be handled by both the UE and P-GW, which would require modifications to the modem/application on the UE side and to the user plane processing module/protocol stack on the P-GW side.

- The default bearer established for IMS signalling may also be used to convey other types of data, such as DNS. As a result, the UE and P-GW must implement additional per-packet detection mechanisms to handle packets with different formats.

Observation 1: The NIDD defined in TS 23.401 is not preferred since the UE lacks IP address, which significantly affects IMS client.

Proposal 1: IMS signalling shall be transported over IP packets.

For text-based simplification and binary-based simplification:

- Based on S2-2508234, the 𝐸𝐶𝑆𝑇𝑠𝑎𝑡𝑇𝑜𝑇𝑒𝑟𝑟, which considers state transition, transmission delay and propagation delay for IMS and EPS, is quite similar between text-based simplification (11.5–15s) and binary-based simplification (~14s). This indicates that from a latency perspective the performance gain of binary-based simplification is limited in GEO NB-IoT NTN scenarios.

- Binary-based simplification is designed to support limited IMS service. To support expanded services such as call-hold, multi-call, or additional services, the encoding needs continuous expansion. This approach has poor scalability and severely restricts service capabilities and flexibility.

- Binary-based simplification has significant implications for CT1, requiring the design of a new encoding scheme specifically for IMS over GEO. This poses a risk of not meeting the R20 timeline.

Proposal 2: The text-based simplification shall be adopted for IMS optimization.

Proposal 3: Whether to adopt binary-based simplification requires CT1's confirmation.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change* * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.1	Interim conclusions for Key Issue #2

Editor's note:	Other interim conclusions for Key Issue#2 is FFS.

The following interim conclusions for Key Issue #2 IMS enhancement for GEO NB-IoT NTN access are made:

-	IMS signalling shall be transported over IP packets.

-	The text-based simplification shall be adopted for IMS optimization.

Editor's note:	Whether to adopt binary-based simplification requires CT1's confirmation.

# * * * * End of change * * * *