3GPP TSG-WG SA2 Meeting #172	S2-2510410

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion update proposal

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#2 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, the architectural conclusion of KI#1 in S2-2509659, and the agreed paper of KI#2 in S2-2509824, this paper provides the conclusion on KI#2 on Authorization and Revocation to Support Sensing Service.

On top of S2-2509824, the following aspects are further clarified:

-	It is the Sensing Function that supporting Sensing authorization functionality;

-	Adding NOTEs for clarifying the split deployment of SCF and SPF;

-	Adding the associating procedures.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.2	Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality;

NOTE 1:	In case of SF splitting into SCF and SPF, the SCF perform the authorisation and revocation of Sensing Service Request

b)	Sensing Function determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any, and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations. (Note 1) |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g., object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| Note 1: the Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM, and stored in Sensing authorization functionality for authorisation of the sensing service request.

d)	Sensing Function can revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

h)	The information for the authorisation of Sensing Service Request is stored in NEF for authorisation of whether the AF is allowed to request Sensing Service and in Sensing Function for authorisation of the sensing request.

NOTE 2:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

The Figure 7.1.2-1 depicts the procedure for authorisation

Figure 7.1.2-1: Procedure for authorization.

1.	The AF sends a Sensing Service Request to the NEF.

2.	The NEF authorises the AF as Sensing Service Consumer as defined in TS 23.502 [10].

3.	The NEF forwards the Sensing Service Request to the Sensing Function.

4.	The Sensing Function determines whether the AF is authorised to perform the Sensing Service Request considering the Sensing Authorization information described in the Table 7.1.2-1.

NOTE 3:	If only part of the Target Service Area is authorized by the Sensing Control Function (e.g. the Target Service Area is partially within the Allowed Sensing Service Area), the subsequent steps continue only with this authorized Target Sensing Service Area.

NOTE 4:	The AF can cancel the previous requested sensing service by sending the associating cancel request.

The Figure 7.1.2-2 depicts the procedure for the revocation of granted authorisation.

Figure 7.1.2-2: Procedure for the revocation of granted authorisation.

1a.	The NEF may decide that the authorisation previously granted to the AF is invalid any more, e.g. due to change of SLA, and send the revocation request to the Sensing Function (or Sensing Control Function in split deployment) and continues with step 2.

1b.	The Sensing Function (may decide that the authorisation previously granted to the AF is invalid any more, for example due to change of SLA, the previously authorized service area is no longer authorized, and the information from others (e.g., load condition) etc., and so the sensing service has to be stopped. In case that AF has been granted for different Sensing Service Requests, the Sensing Function may decide to revoke the authorisation for all requests or some of the requests based on the Target Sensing Service Areas of the requests. For example the AF has been authorised for different Target Sensing Service Areas and for some of them are no more authorised.

2.	Sensing Function cancels all relevant sensing procedures related to the Sensing Service Requests whose authorisation have been revoked in step 1a or 1b.

3.	The Sensing Function sends a Response to NEF if has received the request from NEF (step 1a) or a Notification to the NEF related to the sensing service(s) which has been revoked (step 1b).

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.2 are agreed as the conclusion.

# * * * * End of changes * * * *