3GPP TSG-WG SA2 Meeting #172	S2-2510411

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion update proposal

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#3 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, the architectural conclusion of KI#1 in S2-2509659, and the agreed paper of KI#3 in S2-2509825, this paper provides the conclusion on KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection.

On top of S2-2509825, the following aspects are further clarified:

-	Sensing Function re-selection is not supported in this release;

-	Further clarifies the principles for Sensing entity re-selection;

-	Adding NOTEs for clarifying the split deployment of SCF and SPF;

-	Adding the associating procedures.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.3	Agreed Principles for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed for KI#3:

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and Selection:

a)	The Sensing Function selection is based on NEF's local configuration or NRF query.

b)	The Sensing Function registers itself to the NRF, with its NF profile.

c)	Sensing Function is selected based on at least its supported sensing area.

d)	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

e)	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, including its supported sensing area.

f)	Sensing Function re-selection is not supported in this release.

h)	Sensing Function may re-select one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

i)	Sensing Function may re-select the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, including its supported sensing area.

-	Depending on the change of the Target Sensing Service Area, the Sensing Function may send sensing service requirements to some Sensing Entities, i.e. gNBs in new Target Sensing Service Area, and may cancel the sensing service requirements previously sent to the Sensing Entities, i.e. gNBs in old Target Sensing Service Area.

NOTE 1:	In case of SF splitting into SCF and SPF, the SCF is selected as defined for Sensing Function and the SCF performs the selection of the Sensing Entities;

NOTE 2:	In case of SF splitting into SCF and SPF, the SCF (re-)selects the SPF based on the local information and is implementation-specific;

NOTE 3:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

The Figure 7.1.3-1 depicts the procedure for discovery and selection.

Figure 7.1.3-1: Procedure for discovery and selection.

NOTE 4:	In case of SF is split into SCF and SPF, the SCF participates the procedure as Sensing Function in the call-flow.

0a.	The following configuration may be performed via OAM:

-	NEF is pre-configured with the information for selection of the Sensing Control Function (e.g., SF IP address or FQDN, SF Sensing Service Area);

-	Sensing Control Function is pre-configured with the information for selection of gNB (e.g., gNB IP address or FQDN, Sensing Tracking Area, Sensing Cell);

-	gNB is pre-configure with the SF information to send information for selection;

0b.	The gNB provides its sensing capabilities to Sensing Function;

0c. Sensing Function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile;

1.	The AF sends a Sensing Service Request to the Sensing Function via NEF as described in KI#2 and KI#5.

2.	The NEF selects the Sensing Function(s) based on the local information or via the NRF as described above;

3.	The NEF forwards the Sensing Service Request to the selected SF(s);

NOTE 5:	In case of SF splitting into SCF and SPF, the SCF selects the SPF based on the local information and is implementation-specific;

4.	The SF the selects the Sensing Entities, i.e. gNBs, as described above;

5.	The SF send the sensing service requirements to the selected gNB(s) as described in KI#4 and KI#6.

Figure 7.1.3-2: Procedure for re-selection.

NOTE 6:	In case of SF is split into SCF and SPF, the SCF participates the procedure as Sensing Function in the call-flow.

1.	The SF the re-selects the Sensing Entities;

2.	Depending on the change of the Target Sensing Service Area, the Sensing Function may send sensing service requirements to some Sensing Entities, i.e. gNBs in new Target Sensing Service Area, and may cancel the sensing service requirements previously sent to the Sensing Entities, i.e. gNBs in old Target Sensing Service Area.

NOTE 7:	In case of SF splitting into SCF and SPF, the SCF re-selects the SPF based on the local information and is implementation-specific;

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.3 are agreed as the conclusion.

# * * * * End of changes * * * *