SA WG2 Meeting #172	S2-2510515

17 – 21 Nov, 2025, Dallas, US	revision of S2-2509804

Source:	China Telecom

Title:	[WT#1.3, Non-3GPP Access] WT Scope and KI for non-3GPP access

Document for:	Approval

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the work task scope and key issues for WT#1.3.

**** First Change ****

Annex A.X. WT#1.3

It proposes to add several sub-WTs to address the non-3GPP access scenario and multi-access data connections.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G between 3GPP access and non-3GPP access.

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

NOTE 0:  At minimum, operator voice service over untrusted non-3GPP access needs to be supported. Other services are not excluded.

2.	Study whether and how to support wireline access in 6G System architecture.

NOTE 1:	The work to support wireline access will not depend on BBF or CableLabs;

3.	How to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

4.	How to support Non-Seamless WLAN Offload (NSWO).

NOTE 2:  This bullet is about URSP provided to UE for Non-Seamless WLAN Offload. The Authentication of NSWO is not within the scope for this bullet.

5.	How to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 3:  WT#1.2 aspects related to QoS, Policy, and UP applicable to non-3GPP access are studied under this WT and coordinated with corresponding WTs during the study.

NOTE 4: The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access present.

NOTE 5: Coordination with SA3 for authentication aspects will be needed for this WT.

For the multi-access:

6.	Study whether and how to support the multi-access data connectivity (e.g. traffic steering and switching) between 3GPP access and non-3GPP access for different type of traffic (e.g. IPv4/v6, UDP, TCP, Ethernet) in 6G.

NOTE 6: This bullet will be studied only if the architecture for bullets 1 and 2 is stable, and the study will be conducted on the basis of that architecture, if time allows.

**** Second Change ****

5.X. Key Issue #X: Support of non-3GPP access

This key issue will address the following issues:

For supporting the non-3GPP access:

-	Identify how to support connectivity establishment via non-3GPP access, including the registration and session establishment procedure.

-	Identify how to support service continuity between 3GPP access and non-3GPP access.

For supporting multi-access data connections between 3GPP access and non-3GPP access:

-	Identify how the Core network and the UE can support multi-access data connectivity between 3GPP and non-3GPP accesses.

NOTE 1:	Multi-access connectivity is dependent on the architecture of non-3GPP access and will be studied only if the architecture for non-3GPP access is stable.

**** End of Changes ****