3GPP TSG-WG SA2 Meeting #172	S2-2510561

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI3, Sol#22 update] UE detectable IMS emergency call based on GIBA

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract: Updates solution 22 to support GIBA authentication to reduce the number of interactions between a UE and IMS.

# 1. Introduction/Discussion

TR 23.700-19 solution 22 proposed to skip IMS emergency registration procedure based on the principles of GPRS-IMS-Bundled authentication (GIBA). However, GIBA is the authentication mechanism for IMS registration procedure and can be utilized in the case of GEO satellite access to reduce the number of interactions between the UE and IMS network compared to AKA authentication.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

## 6.22	Solution #22: Supporting UE detectable IMS emergency call on the basis of GIBA authentication

### 6.22.0	High-level solution Principles

This solution comprises the following principles:

-	Based on the GPRS-IMS-Bundled authentication (a.k.a. GIBA) principles described in clauses 5.1.1.4.6, 5.2.2.6 and 5.4.1.2.1E of TS 24.229 [16], UE and IMS network further skip the IMS AKA authentication for IMS emergency registration procedure when GEO access is used.

Editor's note:	Use of GIBA with UE equipped with a USIM needs coordination with SA3.

### 6.22.1	Description

This solution addresses Key Issue #3.

When UE has sufficient credentials to register on the IMS of the serving network, it will perform EPS AKA authentication during EPS emergency attach procedure. The UE also will perform IMS AKA authentication during IMS emergency registration procedure unless the network allows GPRS-IMS Bundled Authentication.

This solution proposes to skip the IMS AKA authentication for IMS emergency registration in case of GEO satellite access to save the emergency call setup time.

### 6.22.2	Procedures

UE

eNodeB

MME

SGW/PGW

HSS

P/S-CSCF

E-CSCF

PSAP

0a. Emergency attach with successful EPS AKA authentication

0b. Emergency PDN Connection establishment

1. Uses GIBA for emergency registration based on RAT type

2. SIP REGISTER

3a. AA-Request (user public identity)

3b. AA-Answer (user public identity, UE IP address)

4. Verify the IP address contained in SIP message based on IP address retrieved from EPC

5. SIP 200 OK

7. UE initiates UE detectable IMS emergency call set up procedure as specified in clause 7.1.1 of TS 23.167

6. Emergency number is detected

Figure 6.22.2-1: UE detectable IMS emergency call Procedure

0.	UE performs EPS emergency attach as specified in TS 23.401 [5], wherein the EPS AKA authentication procedure is successfully done. UE establishes an emergency PDN connection as specified in TS 23.401 [5].

1.	UE determines GIBA is to be used for IMS emergency registration if the RAT type used to access the EPS is related to GEO satellite access.

2.	UE sends emergency registration request (i.e. SIP REGISTER  with an emergency indication) with an indication of supporting IMS emergency registration optimization towards P/S-CSCF.

3.	If the RAT type (obtained from UE or IP-CAN) of the UE is related to GEO satellite access and indication of supporting IMS emergency registration optimization is included, P/S-CSCF determines GIBA is used for UE authentication and requests PCRF to report UE's IP address, based on the user public identity.

4.	P/S-CSCF verifies the UE IP address contained in the SIP REGISTER based on the obtained the UE IP address from EPC as described in clause T.8 of TS 33.203 [28].

5.	After authentication succeeds, the S-CSCF returns SIP 200 OK message to the UE.

6.	Emergency number is detected by the UE based on e.g., local emergency number list.

7.	UE initiates UE detectable IMS emergency call set up procedure as specified in clause 7.1.1 of TS 23.167 [7]. In this case, no IPSec is established between UE and P-CSCF.

NOTE 1:	The negotiation of the capability of omitting IMS AKA authentication for IMS emergency registration can be done during emergency PDN connection establishment, e.g. via emergency PDN connection establishment request and response.

### 6.22.3	Impacts to Services, Entities and Interfaces

UE:

-	Use GIBA for emergency registration if RAT type is related to GEO satellite access.

P-CSCF:

-	Support GIBA for IMS emergency registration based on UE’s RAT Type.

# * * * * End of changes * * * *