3GPP TSG-WG SA2 Meeting #172	S2-2510562

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusions of KI#1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Update of the interim conclusion of KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

# 1.1 Introduction

Interim conclusion on UP voice transport option has been reached in last SA2#171 meeting, which has paved the way for further down-selection on other aspects, i.e. IP or non-IP for voice, one or two PDN connection for signal and voice, dedicated or default bearer for voice, whether to pre-establish the dedicated bearer.

For IP/non-IP issue. In SA2#169 meeting, S2-2504678 pointed out the potential shortage of RoHC which may cause low compress efficiency in GEO satellite scenario, but in SA2#170 meeting, S2-2507021 and S2-2507102 showed vendor’s log to prove the inefficiency caused by ROHC transitions is very limited. Considering the non-IP option has strong dependency on SA4, SA3 and causes more specification and deployment upgrades, e.g. RTP protocol simplification, protocol conversion between simplified RTP/non-IP and full RTP/UDP/IP for IP type PDN connection, TFT configuration for non-IP traffic. Based on the above analysis, it is proposed to reach interim conclusion on voice over IP option.

For one/two PDN connection issue. Two PDN connections option didn’t show further enhancement compared with two bearers in one PDN connection, but will introduce major impact on the existing IMS mechanism including strong UE impact, e.g., new IMS APN to differentiate IMS signalling and voice, new UE logic to trigger the establishment of the second PDN connection, impact on UE and PGW about how to differentiate packets between two PDN connection. Hence, one PDN connection option is proposed.

For dedicated/default bearer issue. The concerns on dedicated bearer mainly focus on the restricted bearer number of NB-IoT, while LS from RAN2 (R2-2507765) understands that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. In order to guarantee the QoS of IMS voice and reduce the standard and deployment impact, it is proposed to reach interim conclusion on dedicated bearer for IMS voice data.

For pre-establish dedicated bearer issue. Since the main cause on the long call setup time is message transfer during call setup procedure, establishing the dedicated bearer in advance can reduce the call setup time. But it will waste bearer resource if pre-establish all the time when there is no IMS call. Hence, it is proposed to pre-establish dedicated bearer when UE has MO IMS call.

Based on the above analysis, it is proposed to reach conclusion with following:

- IMS signalling and voice over IP protocol (including the transfer between UE and PGW/AGW) with Robust Header Compression (RoHC).

- IMS signalling and voice over one IP PDN Connection.

- IMS signalling over default bearer, IMS voice over dedicated bearer.

- UE triggers dedicated bearer for IMS voice to be established when UE has MO IMS call.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Conclusions for Key Issue #1

The following conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	IMS signalling and voice packets shall be transported over the NB-IoT (GEO) user plane, i.e., using DRB and S1-U.

-	The IMS signalling and voice packets shall be transported over IP protocol.

-	The IMS signalling and voice packets shall be transported over one IP PDN connection.

-	The IMS signalling packets shall be transported over default bearer, the IMS voice packets shall be transported over dedicated bearer.

-	The UE can pre-establish dedicated bearer for IMS voice packets transport when it has MO IMS call.

# * * * * End of changes * * * *