SA WG2 Meeting #172	S2-2510568

17 – 21 Nov, 2025, Dallas, TX, USA

Source:	Samsung

Title:	[WT#1.2, Network Slicing] KI proposals for Network Slicing in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Key Issue proposals for Work task 1.2 network slicing

1. Justifications

In SA2#171, WT#1.2 scope on Network Slicing has been agreed as below which includes the simplification of network slicing and improvement of application traffic mapping to network slice(s).

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a.	Identify and address any areas of possible improvement and simplification of network slicing.

b.		Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

NOTE 1:	Coordination between bullet 1b and the WT 1.2.x on Policy is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 2:	Impacts to RAN are possible and coordination with RAN WGs is expected.

Based on the agreed WT scope, we propose the KI for simplified network slicing for 6G based on the following motivation.

Network slicing is one of the most promising technologies in mobile networks that enable a new business opportunity in 5G/6G. While network slicing features have been evolved continuously in 5G, there exist inefficiency that needs to be improved in 6G. For example, in 5G network slicing, support from UE is required for most of the slicing features and how to decouple such dependency on UE need to be investigated in 6G.

We propose the KI for simplified network slicing for 6G based on the following architectural motivation.

Architectural Motivation: Complexity of UE configuration

- Dependencies on UE: In the network, the slice ID (i.e., S-NSSAI) is used to identify and manage the resources of the network that is used to serve a specific type of communication service. In the UE, the slice ID is also used to identify the type of communication services (e.g., Real-time streaming, Low latency, etc.) that it is allowed to use. This caused dependencies between the network and UE for network slicing and consequently, the support from UE is required for the evolutions or introduction of a network slicing feature.

- Many alternative options: Some of the network slicing features have defined too many options. It would be valuable to investigate whether all the options are actually needed or not.

Gap Analysis 1: 5G network slicing design is limited to achieve simplified network slicing

Technical Impact

- Any architectural impact can be investigated for simplification of network slicing

- UE, RAN, NFs can be impacted

**** First Change ****

5.X. Key Issue #X: Simplified Network Slicing for 6G

The key issue will study support for simplified network slicing for 6G:

- Study how to improve the application traffic mapping to network slice(s) to enable an introduction of additional network slicing features without impacting UE

- Identify a set of network slicing features that have multiple options and investigate whether such features can be re-designed with reduced number of options

**** End of Changes ****