SA WG2 Meeting #172	S2-2510614

17 – 21 November, 2025, Dallas, US	(revision of S2-25xxxxx)

Source:	vivo, NTT DOCOMO

Title:	KI#1: conclusions

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to agree the conclusions for KI#1 for FS_EnergySys_Ph2.

1.	Introduction

There are two solutions captured in the TR to enhance the energy consumption calculation formulars in order to minimize the impact of the idle energy consumption of the node.

In solution 10, it proposes to enhance the mathematical approach of energy consumption information for the required granularity defined in Rel-19. For the time interval T, the EIF calculates the per bit energy consumption at the network node by using the ratio of the energy consumption difference to the total data volume difference between the two time intervals. Then, the EIF applies the per bit energy consumption as the coefficient to calculate the energy consumption of the required granularity at the node (i.e., UPF and gNB) over the time interval T. It aims to reduce the impact of the idle energy consumption of the node.

In solution 11, it is proposed that the EIF collects the idle energy consumption of the node based on operator’s configuration. This idle energy consumption of the node can be configured within the EIF. And then the EIF excludes (deducts) the idle energy consumption from the total energy consumption of the node to calculate the energy consumption at the required granularities.

These two calculation examples aim to reduce the impact of the idle energy consumption of the node and improve the robustness of the calculation.

Proposal: It is proposed to agree the new formulas taking idle energy consumption into account as the alternative EC estimation formulas for the required granularity.

2. Text Proposal

It is proposed to agree the following for KI#1 conclusions in TR 23.700-67.

# * * * * First change (all text new)* * * *

8	Conclusions

8.x	Conclusions for KI#1

The following conclusions are agreed for KI#1.

1)	The additional energy related information is exposed by EIF based on agreed principles in clause 7.1.1.

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF depending on SA5 feedback.

3)	The EIF may support the following new formulas for taking idle energy consumption of the node into account as other examples of EC estimation formulas for the required granularity. Whether to adopt the new formulas is left to implementation and operator policy.

-     If the idle energy consumption information of the node is known by the EIF (e.g., via local configuration), this information may be excluded from the total energy consumption of the node to calculate the energy consumption at the required granularities.

-   In order to minimize the impact of the idle energy consumption of the node, the per bit energy consumption information of the network node may be calculated by using the ratio of the energy consumption information difference to the data volume information difference between two time intervals. This per bit energy consumption information may be further used to calculate the energy consumption at the required granularities.

# * * * End of change * * * *