3GPP TSG-SA WG2 Meeting #172	S2-2510682

Dallas, USA, November 17 - 21, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#2, Sol#10: Update to Resolve ENs

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution resolves ENs in solution #10.

1. Introduction

Solution#10 “DO-A Data Delivery and Routing” was included in TR 23.700-30 in SA2#171. This solution proposes NAS based solution for the DO-A data delivery from the AIoT Devices to the AIOTF, as well as subscription based solution (the AF subscribes to the DO-A data delivery to the AIOTF) for the message routing from the AIOTF to the AF (the notification endpoint is provided in the subscribe request).

This contribution proposes some updates to resolve the ENs in the solution.

In Solution#10, there is one EN about message routing for the AIoT NAS DO-A Deivery Request to the AIOTF:

Editor's note:	If the request is routed to a new AIOTF, how the new AIOTF fetches the AIoT device contexts from the old AIOTF is FFS.

In this solution, the AIOTF is involved in the following procedures:

- In “Subscribe to DO-A Data Delivery” procedure, the AIOTF is selected by the NEF to handle the Subscribe Request. In this procedure, the AIOTF neither create nor hold the AIoT device context. It can be any AIOTF which can update AIoT Device Profile Data in the ADM. This AIOTF can be different from the AIOTF which holds the AIoT device context (i.e., the AIOTF which the AIoT device register to).

- In Registration Procedure which is not covered in this solution, it is assumed there is one AIOTF selected by the NG-RAN node which establishes the device context of the AIoT device and allocate the temporary ID (with the AIOTF ID included) to the AIoT device. It can be different from the AIOTF which handles the subscribe request.

- In “DO-A Data Delivery” procedure, it is assumed the temporary ID is used to route the the AIoT NAS DO-A Delivery Request to the AIOTF which holds the AIoT device context. The NOTE 2 clarifies such aspect: “The NG-RAN and the AMF are assumed to route the AIoT NAS DO-A Delivery Request to the AIOTF which holds the AIoT device contexts in the Registration Procedure.”

In our view, like the serving AMF subscribing to the UDM for the subscription data, the serving AIOTF needs to subscribe to the ADM for the AIoT Device Profile Data of the device. Also like the notification of the SDM, once the AIoT Device Profile Data of the device is updated, the ADM needs to notify the AIOTF who has subscribed to the AIoT Device Profile Data of the device.

Also, in our view, in “DO-A Data Delivery” procedure or registration procedure which hasn’t been covered in this solution, if the NAS request is routed to a new AIOTF which is different from the AIOTF which holds the device context, the new AIOTF needs to fetch device context from the old one and updates itself as the serving AIOTF.

[Proposal] Resolve the EN by adding one NOTE in “Subscribe to DO-A Data Delivery” procedure and one NOTE in “DO-A Data Delivery” procedure.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change (All texts are new) * * * *

## 6.10	Solution #10: DO-A Data Delivery and Routing

### 6.10.0	High-level Solution Principles

The solution is based on the following general principles to support DO-A delivery from DO-A capable AIoT devices:

-	The target address(es) of the AIoT device are stored in the AIoT Device Profile data, which are provided in the subscribe request from the AF.

-	The DO-A capable AIoT devices sends DO-A data over AIoT NAS between the AIoT devices and the AIOTF.

-	The AIOTF forwards the DO-A data to the target AF address(es) obtained from AIoT Device Profile data.

### 6.10.1	Description

This solution addresses KI#2 and focuses on the following aspects:

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

This solution assumes the AIoT device has registered towards the network (i.e. the AIOTF) and have the security contexts established between the AIoT devices and the AIOTF.

The AIOTF updates its supported domain information or AIoT Device Permanent ID ranges in the NF Profile to the NRF. The NRF supports the AIOTF discovery based on target device information.

The AF subscribes to the DO-A Data Delivery by sending a Subscribe Request to the AIOTF (optionally via the NEF). After receiving the Subscribe Request to the DO-A Data Delivery, the AIOTF performs AF authorization by checking the AF authorization data. If the AF request is authorized, the AIOTF updates the target address(es) provided in the Subscribe Request into the AIoT Device Profile Data in ADM.

The DO-A capable AIoT devices sends DO-A data over AIoT NAS between the AIoT devices and the AIOTF. For example, the AIoT Device sends NAS DO-A Delivery Request to the AIOTF containing the AIoT Device ID and DO-A data. The AIOTF acknowledges the AIoT Device by sending NAS DO-A Delivery Response. The NAS DO-A Delivery Request and DO-A Delivery Response are security protected.

NOTE:	The form of AIoT Device ID and the security protection of the NAS DO-A Delivery Request/Response are to be addressed by SA WG3.

The AIOTF obtains the target address(es) from the AIoT Device Profile Data in ADM and forwards the DO-A data to the target address(es) together with the AIoT Device Permanent ID.

### 6.10.2	Procedures

#### 6.10.2.1	Subscribe to DO-A Data Delivery

Figure 6.10.2.1-1 Subscribe to DO-A Data Delivery

Figure 6.10.2.1-1 depicts how the AF subscribes to the DO-A data delivery:

1.	The AF sends Subscribe Request to the NEF for DO-A data delivery with the AF ID, target device information (AIoT Device Permanent IDs or Filtering Information) and target address (i.e. notification endpoint of the AF). For AFs in trusted domain, the AF can send Subscribe Request to the AIOTF directly without involving the NEF.

2.	The NEF discovers the AIOTF by providing target device information to the NRF or based on local configuration. The NEF sends the Subscribe Request to the AIOTF for DO-A data delivery with the AF ID, target device information and target address (i.e. notification endpoint of the NEF).

3.	The AIOTF authorizes the AF request to check whether the AF is allowed to subscribe to DO-A delivery, based on the AF authorization data stored in the ADM.

4.	If authorized, the AIOTF updates AIoT Device Profile data with the target address for the target devices. If the target device information from the AF is Filtering Information, the AIOTF needs to update the AIoT device profile data for each device in the group. Alternatively, the AIOTF may send the Filtering Information to the ADM and the ADM is responsible for the updating for each device in the group.

The target address is the notification endpoint of the NEF or the notification endpoint of the AF, depends on which NF sends the Subscribe Request.

NOTE 1:	Once the AIoT Device Profile Data of the device is updated, the ADM notifies to the AIOTF who has subscribed to the ADM of the AIoT Device Profile Data of this device.

5.	The AIOTF sends Subscribe Response towards the NEF indicating the result of the subscribe. In case of failure, failure cause is included. In case of succeed, subscription correlation ID is included.

6.	The NEF sends Subscribe Response to the AF indicating the result of the subscribe. In the case of failure, failure cause is included. In the case of success, subscription correlation ID is included.

#### 6.10.2.2	DO-A Data Delivery

Figure 6.10.2.2-1 DO-A Data Delivery

Figure 6.10.2.2-1 depicts how DO-A data are delivered from the AIoT device towards the AF:

1.	The AIoT Device sends AIoT NAS DO-A Delivery Request to the AIOTF with the Device ID and DO-A data.

For topology 1, the AIoT NAS message is delivered via the RAN reader inside NG-RAN node. The NG-RAN node delivers the NAS message to the AIOTF directly or via the AMF.

For topology 2, the AIoT NAS message is delivered via the UE reader. The UE reader delivers the message to the NG-RAN node and the NG-RAN node delivers the NAS message to the AIOTF via the AMF.

NOTE 1:	The details of AIoT Device ID and the security protection of the NAS DO-A Delivery Request/Response are to be addressed by SA3.

NOTE 2:	The NG-RAN and the AMF are assumed to route the AIoT NAS DO-A Delivery Request to the AIOTF which holds the AIoT device contexts in the Registration Procedure.

NOTE 3:	If the request is routed to a new AIOTF which does not hold the device context, the new AIOTF needs to fetch the device context from the old one, based on the Device ID (e.g., temporary ID containing the AIOTF ID indicating who holds the device context).

2.	The AIOTF authenticates the AIoT device and sends the NAS DO-A Delivery Response to the AIoT Device. The result is included in the response message. In the case of failure, the failure cause is included.

NOTE 4:	The authentication is to be addressed by SA3.

3.	The AIOTF gets the target address(es) of the AIoT device from the AIoT Device Profile data. For each target address, step 4 to step 7 are executed individually.

4.	The AIOTF sends Notify message to the NEF with the AIoT Device Permanent ID and DO-A data, if the target address is the notification endpoint of the NEF. If the target address is the notification endpoint of the AF, the AIOTF sends Notify Request to the AF directly without involving the NEF.

5.	The NEF sends Notify message to the AF with the AIoT Device Permanent ID and DO-A data.

### 6.10.3	Impacts on Services, Entities and Interfaces

The following NFs are impacted:

-	AIOTF:

-	AIoT NAS layer support for DO-A Data Delivery.

-	Notify DO-A data to the target address(es) obtained from AIoT Device Profile Data in ADM.

-	Support subscribe request towards the DO-A data delivery.

-	Perform AF authorization for the subscribe request. If authorized, the AIOTF update the AIoT Device Profile data to include the target address provided in the subscribe request.

-	ADM:

-	Enhance AF authorization data for the subscribe request towards DO-A data delivery.

-	Enhance AIoT Device Profile data for the target address(es).

-	Support subscribe/unsubscribe/notify of the AIoT Device Profile Data.

-	NEF:

-	Support subscribe request towards the DO-A data delivery.

-	Notify DO-A data to the AF.

-	NRF:

-	Support domain information or AIoT Device Permanent ID ranges in the NF Profile of the AIOTF.

-	Support AIOTF discovery based on target device information.

-	AIoT Device:

-	AIoT NAS layer support for DO-A Data Delivery.

* * * End of Change * * * *