3GPP SA WG2 Meeting #172	S2-2510703

Dallas, US, 17th November – 21st November, 2025

# Source:	Huawei, HiSilicon

# Title:	QoS Data Collection Enhancement

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on QoS Data Collection Enhancement

Acronym: TEI20_QDCE

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a Feature

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

As defined in the Table 6.4.2-2 and Table 6.23.2-1 of TS 23.288, the NWDAF can collect data from the UPF, such as packet delay, bit rate, Packet (re)transmission at the QoS Flow level, and N6 Delay and Packet Loss Rate per TCP traffic. However, it lacks application-specific contextual information like service type, APP ID and Flow Info (e.g., bandwidth and latency requirements, packet loss requirements of specific services). For instance, a single QoS Flow may simultaneously carry several different services traffic. By aggregating metrics at the QoS Flow level, the reported data cannot distinguish QoS characteristics of each individual services, limiting the ability of NWDAF to provide dynamic policy assistance, and subsequently the consumer of the analytics is not able to provide differentiated guarantees per service and the network may fail to meet critical service demands.

Furthermore, some key information might be useful for the NWDAF to output the analytics, especially when the data collection from the AF regarding QoE is not available, for example:

- the application start time and active time, which can be used by the NWDAF to accurately analyze the QoE of the corresponding application.

- the N3 delay, which reflects the current performance of the network and contributes to the QoE of the application;

- the bit rate and bandwidth of the application, which can be used to evaluate whether the requirements of a specific application can be fulfilled by the network;

- the uplink and downlink of the data volume, which reveals the user behavior and activity patterns, e.g., a huge amount of downstream data indicates that the user is downloading or watching videos, which may require a large downstream bandwidth;

- number of packets, which can be used to justify the contribution of packet loss rate to the QoE;

The following Table lists several metrics to be reported from the UPF for NWDAF to provide precise analytics and prediction on the QoE and QoS parameters.

| Information | Description |
| --- | --- |
| Application ID | Identify the application for which the data is reported. |
| Flow Description | Identify the service flow for the application. |
| Application Start Time | Time of the application start. |
| Application Active Time | The time stamp of the application when it is switched to active, or the start and end time of the activity per application. |
| N3 Delay | Describe average, maximum and minimum delay between the UPF and the RAN. |
| Bandwidth | Describe the average, maximum, minimum of uplink and downlink bandwidth |
| Bit Rate | Describe the average bit rate of uplink and downlink. |
| Volume | Describe the data volume (uplink, downlink and/or overall data volume). |
| Number of Packets | Describe the number of packets exchanged (uplink, downlink and/or overall data volume). |

4	Objective

The objective of this work item:

WT-1: UPF data collection enhancements for the QoS and Policy Assistance analytics, i.e. support of new event of UPF to expose application-level and flow-level QoS metrics to enable the NWDAF to accurately calculate and predict the QoE of the requested QoS parameter set(s).

TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 |  | 0.5 | No | None |

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Support of new event of UPF for QoS data collection enhancement | SA#112 (June 2026) |  |
| TS 23.288 | Enhancements on the input data of QoS and Policy Assistance Analytics | SA#112 (June 2026) |  |

6	Work item Rapporteur(s)

xxx

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

China Mobile

Huawei

HiSilicon