3GPP SA WG2 Meeting #172	S2-2510703a

Dallas, US, 17th November – 21st November, 2025

Source:	Huawei, HiSilicon

Title:	Discussion for TEI20 WID on QoS Data Collection Enhancement

Document for:	Discussion

Agenda Item:	30.1

Work Item / Release:	TEI20_QA / Rel-20

Abstract of the contribution: This paper discusses a new TEI20 WID proposal on QoS Data Collection Enhancement.

- Introduction

In 5G networks, the UPF is the cornerstone for achieving QoS guarantees, responsible for traffic classification, scheduling, and resource allocation, and it can interact with other network functions such as NWDAF to assist data analytics. Due to the diversity and complexity of the services, so far the data specified to be reported to the NWDAF are not sufficient to support the NWDAF in performing high-precision analysis and prediction of the QoE and the QoS parameters which can satisfy the requested QoE.

- Discussion

As defined in the Table 6.4.2-2 and Table 6.23.2-1 of TS 23.288, the NWDAF can collect data from the UPF, such as packet delay, bit rate, Packet (re)transmission at the QoS Flow level, and N6 Delay and Packet Loss Rate per TCP traffic. However, it lacks application-specific contextual information like service type, APP ID and Flow Info (e.g., bandwidth and latency requirements, packet loss requirements of specific services). For instance, a single QoS Flow may simultaneously carry several different services traffic. By aggregating metrics at the QoS Flow level, the reported data cannot distinguish QoS characteristics of each individual services, limiting the ability of NWDAF to provide dynamic policy assistance, and subsequently the consumer of the analytics is not able to provide differentiated guarantees per service and the network may fail to meet critical service demands.

Furthermore, some key information might be useful for the NWDAF to output the analytics, especially when the data collection from the AF regarding QoE is not available, for example:

- the application start time and duration, which can be used by the NWDAF to accurately analyse the QoE of the corresponding application.

- the N3 delay, which reflects the current performance of the network and contributes to the QoE of the application;

- the bit rate and bandwidth of the application, which can be used to evaluate whether the requirements of the a specific application can be met by the network;

- the uplink and downlink of the data volume, which reveals the user behavior and activity patterns, e.g., a huge amount of downstream data indicates that the user is downloading or watching videos, which may require a large downstream bandwidth;

- number of packets, which can be used to justify the contribution of packet loss rate to the QoE;

The following Table 2.1 lists several metrics to be reported from the UPF for NWDAF to provide precise analytics and prediction on the QoE and the QoS parameters.

Table 2.1 application related information and QoS metrics reported from UPF

| Information | Description |
| --- | --- |
| Application ID | Identify the application for which the data is reported. |
| Flow Description | Identify the service flow for the application. |
| Application Start Time | Time of the application start. |
| Application Active Time | The time stamp of the application when it is switched to active, or the start and end time of the activity per application. |
| N3 Delay | Describe average, maximum and minimum delay between the UPF and the RAN. |
| Bandwidth | Describe the average, maximum, minimum of uplink and downlink bandwidth |
| Bit Rate | Describe the average bit rate of uplink and downlink. |
| Volume | Describe the data volume (uplink, downlink and/or overall data volume). |
| Number of Packets | Describe the number of packets exchanged (uplink, downlink and/or overall data volume). |

It is proposed to create a work item (TEI20_QDCE) to define a new UPF event for QoS Metrics, enabling the reporting of application specific QoS metrics to the NWDAF for precise analytics, to assist the consumer such as PCF to adjust the QoS and policy accordingly.

- Proposal

Based on the discussion above, the WID and the draft CRs were submitted:

1) S2-250xxxx: New WID on QoS Data Collection Enhancement (TEI20_ QDCE)

2) S2-250xxxx: Draft CR to QoS Data Collection Enhancement in TS 23.502

3) S2-250xxxx: Draft CR to QoS Data Collection Enhancement in TS 23.288