3GPP TSG-WG SA2 Meeting #172	S2-2510703b

# Dallas, US, 17th November – 21st November, 2025	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev |  | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

X

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of new event of UPF for QoS data collection enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_QDCE |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As defined in the Table 6.4.2-2 and Table 6.23.2-1 of TS 23.288, the NWDAF can collect data from the UPF, such as packet delay, bit rate, Packet (re)transmission at the QoS Flow level, and N6 Delay and Packet Loss Rate per TCP traffic. However, it lacks application-specific contextual information like service type, APP ID and Flow Info (e.g., bandwidth and latency requirements, packet loss requirements of specific services). By aggregating metrics at the QoS Flow level, the reported data cannot distinguish QoS characteristics of each individual services, limiting the ability of NWDAF to provide dynamic policy assistance, and subsequently the consumer of the analytics, such as PCF, is not able to provide differentiated guarantees per service and the network may fail to meet critical service demands. Besides, considering the performance issues caused by frequent data reporting, Nupf_EventExposure_Subscribe from NWDAF should provide thresholds for bandwidth and latency reporting, to reduce the impact on UPF performance due to constant report of large amount of data, while still allow the UPF to report in case of specific events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new event of UPF to report QoS related data at application and flow level. - Add thresholds of bandwidth and latency in Nupf_EventExposure_Subscribe service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF cannot collect more finer granularity QoS metrics from the UPF, which would limit the ability of NWDAF to provide more precise and dynamic policy assistance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.1, 5.2.8.3.1, 5.2.26.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ********************* First Change *********************

4.15.4.5.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see clause 5.2.26.2) is used for UPF data collection.

The list of NF consumer which may receive UPF event notifications is defined in clause 5.8.2.17 of TS 23.501 [2].

To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. This is further defined in clause 5.8.2.17 of TS 23.501 [2].

The UPF event exposure events are described in clause 5.2.26.2. In this Release of the specification, the following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information.

-	User DataUsage Measures. This event provides information of user data usage of the User PDU Session.

-	User DataUsage Trends. This event provides statistics related to user data usage of the User PDU Session.

-	QoS Metrics. This event provides QoS metrics information.

A consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). TS 23.501 [2] describes the QoS monitoring parameters that can be measured by means of QoS monitoring and how to enable the measurements for QoS flows. When the Subscription request for QoS monitoring event indicates that it is for the QoS Flow associated with the default QoS rule, based on local configuration the subscription request may trigger SMF to enable QoS monitoring.

NOTE 1:	Packets from multiple applications may share the QoS Flow associated with the default QoS Rule which may diminish the relevance of some measurements like data rate.

The subscription to QoS monitoring event can target the QoS flows bound to an application by including an Application Identifier. In this case, at subscription request and/or when the PCC rules change, SMF identifies the active PCC Rule that includes a DataCollection_ApplicationIdentifier matching that Application Identifier. SMF enables this consumer (e.g. NWDAF) to receive the QoS Monitoring reports enabled by that PCC Rule. The consumer may indicate that it can receive QoS Flow Performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the Application Identifier (that is, if no PCC rule is identified). In this case the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated with the default QoS rule and include the Indication of QoS Flow associated with the default QoS Rule (see clause 5.8.2.18 of TS 23.501 [2]). The UPF will then include the Indication of QoS Flow associated with the default QoS Rule in the Nupf_EventExposure_Notify service operation when sending reports. Otherwise, the SMF may accept the request and indicate in the response that reporting will be activated when the measurements are enabled by a PCC rule or the SMF may reject the subscription request for that Application Identifier.

NOTE 2:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

A consumer of UPF event exposure such as NWDAF/AF/NEF may subscribe to User Data Usage events (i.e. User Data Usage Measures and User Data Usage Trends) and QoS Metrics directly to UPF (under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]) or via SMF, and UPF sends the event notifications directly to this consumer. For these events, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request targets the traffic matching Event Filter Information (details are described in clause 5.2.26.2.1) for a user's PDU Session or for each PDU Session served by the UPF. The subscription request may indicate the granularity requested, that is whether the measurement reports should be provided per data flow, per application, or per PDU Session.

If the event notification can be delayed, i.e. delay tolerant, Reporting suggestion information is included. The Reporting suggestion information includes Report urgency and Reporting time information. Reporting urgency information indicates whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting time is also provided, which defines the last valid reporting time and UPF shall report the detected event before the last valid time.

If a consumer subscribes to an UPF event via the SMF including an AoI ("Area of Interest"), the SMF starts the subscription to the UPF only when the UE is located in the requested AoI. When the UE leaves the AoI, the SMF stops the subscription on the UPF. The SMF may subscribe to AMF about UE moving in or out of an AoI for this purpose. Clause 4.15.4.5.4 describes an alternative procedure for subscription via SMF to UPF event exposure service related with AOI.

If a consumer of UPF event exposure include a Remaining data reporting indication, UPF should send the collected data to the consumer at N4 session release (details are described in clauses 4.15.4.5.6 and 4.15.4.5.7).

Table 4.15.4.5.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF (NOTE 4) |
| --- | --- | --- |
| UE identification (NOTE 5) | Y | Y |
| Any UE | Y | Y |
| DNN | Y | Y (NOTE 6) |
| S-NSSAI | Y | Y (NOTE 6) |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE 3) | Y | N |
| UPF Id (NOTE 3) | Y | not applicable |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering Information (NOTE 1), (NOTE 2) | Y | Y |
| Reporting suggestion information | Y | Y |
| Remaining data reporting indication | Y | Y |
| Subscription termination reporting indication | N | Y |
| NOTE 1: Application ID and Traffic Filtering Information are exclusive. NOTE 2: This parameter does not apply to QoS monitoring event. NOTE 3: This parameter is used to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50]. NOTE 4: This column is not applicable to events where SMF-UPF interactions are using PFCP (e.g. QoS Monitoring event). That interaction is described in clause 5.8.5 of TS 23.501 [2]. NOTE 5: Input parameter when the target is a UE. When the target is a UE, the SUPI is the the UE identification input to SMF. For IP PDU session type, the UE identification input to the UPF shall be the UE IP address associated with the PDU session. For non-IP PDU session types, when the UPF is allowed by local SMF configuration to receive the SUPI associated with a N4 session, the UE identification input to the UPF is the SUPI. UPF event Exposure targeting a UE is not supported for non-IP PDU session types, when the UPF is not allowed by local SMF configuration to receive the SUPI associated with a N4 session. NOTE 6: In a deployment where UPF reporting filtered per DNN and /or S-NSSAI is expected, the SMF should provide UPF with the necessary information over N4. If the SMF is not configured to provide UPF with this information then such UPF reporting filtering is not possible. |  |  |

# ********************* Second Change *********************

5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics from SMF as specified in TS 23.288 [50] and from UPF as specified in clause 4.15.4.5;

-	Allow the EIF to collect data from SMF as specified in clause 5.51.2 of TS 23.501 [2] and in clause 4.29.2;

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	RAT Type.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

-	Candidate DNAI(s) for the PDU Session.

-	Change of common EAS.

-	Simultaneous Connectivity failure: a notification indicating that the simultaneous connectivity over the source and target PSA at edge relocation is not possible (as described in clause 6.3.4 of TS 23.548 [74])

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in clause 5.6.7 of TS 23.501 [2].

NOTE 2:	The UP path change notification includes DNAI to represent the change of satellite identifier in case of UE-Satellite-UE (UE-SAT-UE) communication.

NOTE 3:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

-	QoS Monitoring: the event notification may contain the QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2]. Implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) may trigger SMF to send this event notification.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session. For MA PDU Session the Change of Access Type may include two Access Type information that the user is currently using.

-	Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session and may indicate:

-	whether local traffic offload is possible, i.e. mobility of the PDU session either towards HPLMN or towards a VPLMN where HR-SBO is supported and allowed; and

-	DNN and S-NSSAI of HPLMN.

-	Change of Satellite backhaul category; The event notification contains the new Satellite backhaul category for the PDU session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent whenever a new QoS Flow is established and contains:

-	If the Target of Event Reporting is a PDU Session and the QoS Flow is associated with this PDU Session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS Characteristics attributes, and optionally Maximum Packet Loss Rate, GFBR and MFBR corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is a SUPI and the PDU Session is associated with this SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS Characteristics attributes, and optionally Maximum Packet Loss Rate, GFBR and MFBR corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id and the PDU Session is associated with this Internal-Group-Id (i.e. the PDU Session belongs to a UE belonging to this Internal-Group-Id), both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS Characteristics attributes, and optionally Maximum Packet Loss Rate, GFBR and MFBR corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is any UE, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI or QoS Characteristics attributes, and optionally Maximum Packet Loss Rate, GFBR and MFBR corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	QoS Flow change: The event notification is sent whenever application traffic is bound to an existing QoS Flow or the QoS parameters of the QoS Flow that the application traffic is bound are modified. The Target of Event Reporting may be SUPI, Internal-Group-Id, or any UE. Related event notifications contain parameters as specified for the QFI allocation event.

-	QFI deallocation: The event notification is sent whenever the QoS Flow is released. The Target of Event Reporting may be SUPI, Internal-Group-Id, or any UE. Related event notifications contain parameters as specified for the QFI allocation event.

NOTE 4:	When the consumer NF is the NWDAF, the QFI allocation, QoS Flow change, and QFI deallocation events are used to collect data for analytics as specified in TS 23.288 [50].

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [69]).

-	PDU Session status (activated, deactivated).

NOTE 5:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE session behaviour trends (see clause 4.15.4.3);

-	UE communications trends (see clause 4.15.4.3);

-	UP with redundant transmission: the event notification indicates if redundant transmission (see clause 5.33.2.2 of TS 23.501 [2]) has been activated or not for the PDU session;

-	User Data Usage Measures (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of SMF. Consumer receives the events directly from UPF;

-	User Data Usage Trends (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of the SMF. Consumer receives the events directly from UPF; and

-	QoS Metrics (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of the SMF. Consumer receives the events directly from UPF.

-	Service Signalling characteristics: The event notification includes Request type and number from NF, Number of receiving Session Report from UPFs, State transition information, Timer information, Unexpected operational status indicator, etc. from SMF for Signalling Storm Analytics as specified in clause 6.22.2 of TS 23.288 [50].

-	Usage data per gNB and (I-)UPF: the notification reports data specified in clause 5.51.2 of TS 23.501 [2] as part of the Energy Consumption information collection defined in clause 4.29.2.

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the events QoS Monitoring, User Data Usage Measures, User Data Usage Trends and QoS Metrics are used to collect data from UPF for analytics as specified in clause 4.15.4.5 and in TS 23.288 [50]. SMF conveys the subscription to UPF on behalf of the NWDAF or the SMF conveys the subscription to I-SMF on behalf of the NWDAF and the I-SMF conveys the subscription to UPF on behalf of SMF.

The consumer NF may request to subscribe the UPF exposure events (including event ID of exposed UPF event of QoS monitoring, User Data Usage Measures, User Data Usage Trends and QoS Metrics) via SMF indirectly by Nsmf_EventExposure. After receiving this subscription request, the SMF does a third-party subscription onto UPF on behalf of this consumer. The consumer should also provide the subscribed UPF event to SMF.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| Candidate DNAI(s) has changed | None |
| PDU Session Release | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| PDU Session Establishment | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QoS Monitoring | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = AoI, value = AoI1> <Parameter Type = UPF Id, value = UPF Id1> <Parameter Type = DNAI, value = DNAI1> |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| QoS Flow change | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| QFI deallocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| UP with redundant transmission | <Parameter Type = DNN, Value = DNN1> |
| User Data Usage Measures | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> <Parameter Type = SSID/BSSID, Value = SSID/BSSID1> |
| User Data Usage Trends | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> |
| QoS Metrics | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> |
| Usage data per gNB and (I-)UPF (NOTE 3) | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) |
| NOTE 1: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. NOTE 2: These Parameters are exclusive and only one of them can be provided. NOTE 3: used by EIF to collect data from SMF as specified in clause 5.51.2 of TS 23.501 [2] |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

An SMF receiving Nsmf_EventExposure Subscription request to subscribe (e.g. to UPF event exposure) for "Any UE" does not consider PDU sessions for which it is acting as I-SMF when it selects the PDU session(s) it has to consider.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 6:	Acknowledgement to a UP path change event notification is further described in clause 5.6.7 of TS 23.501 [2].

# ********************* Third Change *********************

5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information and can be used for direct event notification of QoS Monitoring reports to a different NF than the SMF as defined in clause 5.8.2.18 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in clause 5.8.5.11 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result for the QoS monitoring parameter(s) defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay.

-	Indication of QoS Flow associated with the default QoS Rule (if requested by SMF, see clause 4.15.4.5.1).

-	User Data Usage Measures or User Data Usage Trends. These events provide information about traffic matching Event Filter Information (see below) for a user's PDU Session or for each PDU Session served by the UPF. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. Other direct consumers can subscribe directly to UPF under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]. The subscription request contains the Type of Measurement that is being requested, the Target of Event Reporting and the Event Filter Information (which describes the target traffic) and the required Granularity of Measurement. The Target of Event Reporting can be either a specific PDU Session or "any UE". The Event Filter Information may contain one or more of the following parameters: DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information. The Granularity of Measurement can indicate per data flow, per application, or per PDU Session. A combination of the information listed below for User Data Usage Measures can be requested.

For User Data Usage Measures, the event notification may contain following event related information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurement.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurement.

-	Application related Information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF).

For User Data Usage Trends, the event notification may contain following event related information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurement.

For User Data Usage Measures and User Data Usage Trends, if the subscription is targeting any UE, the UPF shall perform the requested measurements for every PDU Session that matches the Event Filter Information and send the information per PDU Session, including the DNN and S-NSSAI and optionally, the sampling ratio achieved by the UPF. When the subscription requests that information is provided per data flow, the notification includes the Packet Filter Set and the Applications Identifier, if available.

NOTE 1:	If the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact of the per PDU Session measurement.

NOTE 2:	The UPF can combine information for different PDU Sessions into one notification message (see TS 29.564 [93]).

-	QoS Metrics. This event provides QoS metrics in finer granularity, i.e. application and flow level, and can be subscribed by NWDAF for Data collection from UPF for QoS and Policy Assistance Analytic (see TS 23.288 [50]) as described in clause 4.15.4.5.

If the comsumer is NWDAF, the subscription shall include thresholds for bandwidth and latency respectively, and the event notification may contain following information as defined in clause 6.23.2 of TS 23.288 [50]:

-	Application ID and/or IP Packet Filter set to indicate applications and/or flows.

-	QoS metrics, e.g., packet loss rate, bandwidth and bit rate.

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.5.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in clause 5.28 of TS 23.501 [2].

-	UE NAT mapping. This event supports the NF consumer to subscribe for obtaining the NATed UE public IP address and port number as defined in clause 5.8.2.17 of TS 23.501 [2]. The Target of Event Reporting is set to UE (private) IP address and Event Filter Information can include one or more of the following parameters: IP address of the remote end, port number of the remote end DNN and S-NSSAI and the IP domain.

NOTE 3:	Port number of the remote end is needed when symmetric NAT is used and UE could possibly have connections with multiple ports of the same remote end IP address.

-	Handling of Payload Headers. This event reports that a Handling of Payload Header requested action has been enforced by UPF (i.e. an occurrence of the requested action) as defined in clause 5.6.17 of TS 23.501 [2].

For this event, UPF reports which action has been enforced.

Handling of Payload Headers event results out of an implicit subscription received by UPF over N4 triggered by an AF subscription request to PCF via the Npcf_SMPolicyControl service or by the decision in SMF to request notifications of the enforcement of the Handling of Payload Headers instructions provided to UPF.

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF as described in clause 5.8.2.17 of TS 23.501 [2]. The UPF collects the data according to the event ID and exposes the related information directly regardless of whether the subscription has been relayed by the SMF.

# ********************* End of Changes *********************