3GPP TSG-WG SA2 Meeting #172	S2-2510703c

# Dallas, US, 17th November – 21st November, 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR |  | rev |  | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements on the input data of QoS and Policy Assistance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_QDCE |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As proposed in TEI_20 QDCE, reporting QoS metrics at application and flow level is benefitial for more precise analytics and prediction of QoE and QoS parameters by the NWDAF. Although some data can be collected from other NFs and OAM, UPF as the NF that directly processes service data flow, can report more real-time and accurate data to NWDAF directly. In addition, collecting related data from the source NF directly can be more efficient and simplify operations such as data formatting and processing performed by the NWDAF. Therefore, new data related to application and flow level QoS metrics are added into table 6.23.2-1 of clause 6.23.2 for UPF data collection, to enhance NWDAF for QoS and Policy Assistance Analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding QoS metrics from UPF as input data in table 6.23.2-1. - Modifying description of N6 delay to support average, maximum and minimum value. - Moving loss packet rate into the QoS metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF cannot collect more finer granularity QoS metrics from the UPF, which would limit the ability of NWDAF to provide more precise and dynamic policy assistance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ********************* First Change *********************

6.23.2	Input Data

The NWDAF supporting QoS and policy assistance analytics shall be able to collect information from AF, OAM and 5GC NFs.

The following specified input data might be reused by QoS and policy assistance analytics:

-	The input data defined for Observed Service Experience related network data analytics in clause 6.4.2, e.g. the information of the application, Service Experience and QoE metrics.

-	The input data collected from the OAM for Network Performance analytics as defined in clause 6.6.2, e.g. the RAN status, load and performance per Cell Id for the traffic type of interest and in the Area of Interest.

In addition, more information collected by the NWDAF from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.23.2-1 and from OAM in Table 6.23.2-2.

NOTE:	The NWDAF may also collect the Observed Service Experience Analytics from other NWDAFs, based on NWDAF implementation.

Table 6.23.2-1: Input data from 5GC NF related to QoS and Policy Assistance information

| Information | Source | Description |
| --- | --- | --- |
| QFI allocation, QoS Flow change, or QFI deallocation events (1…max) | SMF | SMF provides such events (see clause 5.2.8.3.1 of TS 23.502 [3]). |
| > Used QoS parameter combination | SMF | The QoS parameter combination that has been applied by the SMF, including the following parameters as defined in Clause 5.7 of TS 23.501 [2]: - 5G QoS Identifier (5QI); - and if available: - 5G QoS characteristics; - Guaranteed Flow Bit Rate (GFBR) - UL and DL; and - Maximum Flow Bit Rate (MFBR) - UL and DL; and - Maximum Packet Loss Rate - UL and DL. |
| > Traffic descriptor | SMF | One of Application Identifier, IP Packet Filter Set or Ethernet Packet Filter Set |
| > S-NSSAI | SMF | Slice used to transport the QoS Flow(s). |
| > DNN | SMF | DNN associated with the QoS Flow(s). |
| > DNAI | SMF | Identifies the access to the DN that the PDU Session is connected to. |
| UE identifier | SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| RAT Type | SMF | RAT Type used for the UE. |
| Time stamp | SMF, AMF | The time stamp associated to the collected data. |
| N6 Delay (NOTE 1) | UPF | The average, maximum and minimum of the observed N6 delay between the UPF and measurement endpoint in the DN, as defined in clause 5.8.2.23 of TS 23.501 [2]. |
| QoS metrics | UPF | The QoS metrics at application and/or IP flow level, including the following information: - Application ID and IP Packet Filter set, as defined in clause 5.8.2 of TS 23.501 [2]; - QoS metrics at both levels: - N3 delay, i.e. average, maximum, minimum of the packet delay measurement on N3 between NG-RAN and PSA UPF, as defined in clause 5.4.7 of TS 28.552 [8]; - Bandwidth, i.e. average, maximum, minimum of bandwidth for UL and DL; - Packet Loss Rate for UL and DL (NOTE 2); - Bit Rate, i.e. the observed average bit rate for UL and DL; - and QoS metrics at application level: - Volume, data volume measured (UL, DL and/or overall) and number of packets measured (UL, DL and/or overall); - Start Time, the start time of traffic for the application and/or flow; - Active Time, the time stamp of the application when it is switched to active, or the start and end time of the activity per application. |
| NOTE 1: Whether and how N6 delay (independent of QoS Flow) is considered to derive the analytics in comparison to delay between UE and UPF (dependent on the QoS Flow) is left to implementation based on the operator's policy. NOTE 2: The Packet Loss Rate is applicable to the TCP traffic for which the UPF is able to provide the information. |  |  |

For the analysis of a QoE that must meet the provided Packet Delay Budget, the following input is collected to enable the latency analysis between UE and PSA UPF.

Table 6.23.2-2: Input data from OAM related to Packet Delay between UE and Anchor UPF

| Information | Source | Description |
| --- | --- | --- |
| UL/DL GTP packet delay | OAM | Average UL/DL packet delay measurement on N3 between NG-RAN and PSA UPF, as defined in clause 5.4.7 of TS 28.552 [8]. |
| Delay in RAN | OAM | Average Uplink and Downlink packet transmission delay over the air interface, as defined in clause 5.1.1.1 of TS 28.552 [8]. |

*** End of change ***