SA WG2 Meeting #172	S2-2510772

17 - 21 Nov, 2025, Dallas, USA

Source:	Huawei, HiSilicon, Ericsson

Title:	Update for KI#2 Interim Agreements and Conclusions

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for AIML_CN KI#2 to remove UPF as a consumer of NWDAF. As an alternative, configure some pre-defined N4 rules for the UPF to act based on the analytics of the abnormal traffic received directly from the NWDAF.

# Discussion

Some interim agreement principles are agreed for KI#2 in TR 23.700-04. However, regarding UPF as consumer, we identified some problems as following:

- UPF is a user plane function, having no subscriber and application information and no ability to make decision on its own to influence the traffic handling, and all its actions are controlled by the SMF via N4 rules, which is further controlled by the PCF, via PCC rules.

- Requesting UPF to be a consumer of the NWDAF, the UPF needs to behave like an SMF, and figure out the responsible NWDAF to which to subscribe, e.g. interacting with NRF with the selection criteria, and populate the subscription request accordingly (with information it does not have or is not able to make decision, such as AoI, the S-NSSAI/DNN, time considerations, and so on).

- In the interim agreement as described in clause 7, both the SMF and PCF can be consumers of NWDAF. They have all the information for the subscribers and applications, and the status of all the related UPFs, and the SMF and the PCF (via SMF) can provide policies to the UPF based on the analytics subscribed from the NWDAF, taking into account the subscriber and application information, and network status and so on, and the UPF acts based on the instruction from the SMF. If allowing UPF as consumer of the NWDAF, even enabling UPF to be kind of policy decision function, there would be duplicate subscriptions from SMF/PCF and UPF to NWDAF, which then may generate conflict decision between UPF and SMF/PCF.

Based on the above identified problems, apart from the complexity introduced by the UPF as consumer of NWDAF, waste of resources and potential conflict with the policy from the SMF/PCF, we don’t see the benefit to make UPF become a control plane NF and behave like SMF, so propose to remove UPF as consumer from the interim agreement.

As an alternative, it can be considered to configure some pre-defined N4 rules for the UPF to act based on the analytics of the abnormal traffic. The SMF can subscribe to NWDAF on the analytics, such as the type of abnormal traffic, which can be notified by the NWDAF directly to the UPF as activation and deactivation of the pre-defined N4 rules.

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

****************************** Option 1 (preferred)**************************

*** First change ***

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288) for input data filtering to UPF.

#### 7.1.2.2	Agreed Principles for Use Case #1

- The service consumer of the analytics service may be SMF, PCF, OAM, and AF. The consumer may take the output Analytics into account.

- To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

- To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to enforce the downlink traffic suppression (e.g., selective packet dropping or rate limitations) or shape abnormal traffic, to enforce bandwidth limitations.

NOTE Y: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor).

- The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE Z: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

*** Second change ***

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-  For UC1, the input from AF, possibily via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

*** End of the changes ***

****************************** Option 2 **************************

*** First change ***

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288) for input data filtering to UPF.

#### 7.1.2.2	Agreed Principles for Use Case #1

- The service consumer of the analytics service may be SMF, PCF, OAM, and AF. The consumer may take the output Analytics into account.

- To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

- To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to enforce the downlink traffic suppression (e.g., selective packet dropping or rate limitations) or shape abnormal traffic, to enforce bandwidth limitations.

- The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- The operators may preconfigure some pre-defined N4 rules in both the SMF and the UPF for the handling of the abnormal traffic. In this case, the SMF may subscribe to NWDAF services on the new Analytics ID on abnormal data packets patterns on behalf of the UPF, and the NWDAF can send directly the analytics to the UPF for activation and deactivation of the per-defined N4 rules in the UPF for the abnormal data packet handling, such as enforcing the downlink traffic suppression (e.g., selective packet dropping or rate limitations) or adjust the packet processing resources. The condition to activate or deactivate the N4 rule in the UPF depends on the received Analtyics.

- The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE Z: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

*** Second change ***

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-  For UC1, the input from AF, possibily via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

*** End of the changes ***