3GPP SA WG2 Meeting #172	S2-2510774

Dallas, USA, 17-21 November 2025

# Title:	[Draft] LS on support of multiple PLMNs for IMS emergency over NB-IoT NTN

# Response to:	-

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	SA2

To:	SA1, RAN2

Cc:	CT1

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 is in the process of studying support for IMS emergency calls over NB-IoT under the SID FS_5GSAT_Ph4_ARC. SA2 observes that in WB-EUTRAN the indication for support of IMS emergency calls in SIB1 (emergencyCallSupport flag) is per cell. This means that, in case of RAN sharing, all PLMNs sharing the same cell have to either support or not support IMS emergency calls. In TN scenarios the above may not be an issue since typically all PLMNs belong to the same country and therefore have to comply to same regulatory requirements. Nevertheless, for NB-IoT NTN it is possible that multiple country specific PLMN-IDs (from different countries), or country specific and non country specific PLMN-IDs share the same NTN cell. Thus, SA2 would like to ask whether it is possible that the indication for support of IMS emergency calls, that will have to be defined in rel.20, can be made PLMN specific and allow each individual PLMN sharing the cell to indicate to UE whether IMS emergency calls are supported or not.

SA2 has not made final conclusions on this topic and overall for SID FS_5GSAT_Ph4_ARC.

2. Actions:

To SA1:

ACTION: 	SA2 would like to ask SA1 whether in case of NTN RAN sharing, the support of Emergency calls indicated in SIB should allow different indication of support between different PLMNs sharing the same cell.

To RAN2:

ACTION: 	SA2 would like to ask RAN2 whether they see any issues to make the indication of support for IMS emergency calls PLMN specific in the new SIB structure that will have to be potentially defined for NB-IoT NTN

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #173		9-13 February 2026	Goa, India

TSG-SA2 Meeting #173		13-17 April 2026	EU