SA WG2 Meeting #171	S2-2509282

Wuhan, China, 13 – 17 October 2025	                  (revision of S2-2508489)

Source:	China Mobile

Title:	KI#3: New Sol: Non-UE detectable IMS Emergency Calls for UE via NB-IoT GEO Access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#3.

# 1	Discussion

For emergency calls over GEO NB-IoT NTN access, it is also necessary to consider meeting SA1’s IMS voice call setup time KPI over GEO satellites.

This contribution focuses on non-UE-detectable IMS emergency session procedures during GEO NB-IoT NTN access. By concentrating on the non-UE-detectable case, the proposed solution addresses the limitations of NB-IoT for emergency services while minimizing call setup delays in high-latency GEO satellite environments.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  |  | x |  |

* * * Next Change * * *

6.X	Solution #X: <IMS Emergency Service for NB-IoT NTN Access>

6.X.1	High level principles

This solution resolves Key Issue #3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC,  the following  high level principles are included in this solution:

- 	For Non-UE detectable IMS emergency sessions over GEO NB-IoT NTN access, signalling interactions between the UE and the network shall be minimized in order to reduce additional latency introduced by GEO satellite propagation delay.

-	The IMS core shall be capable of identifying emergency call attempts initiated by a UE without emergency indication (non-UE detectable) and handle such calls as emergency sessions.

- X.2	Description

This solution resolves Key Issue #3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

As specified in TS 23.167, emergency call services are categorized into UE Detectable Emergency Session and Non-UE Detectable Emergency Session based on UE capabilities.

For Non-UE Detectable Emergency Session,  (i.e. the UE initiates the call as a normal IMS call without indicating emergency), two approaches are considered to handle the call in the network:

Option 1: IMS notifies UE of emergency . In this approach (aligned with existing procedures in TS 23.167), the IMS core identifies that the dialed number is an emergency number and informs the UE that the session is an emergency. For example, the P-CSCF can respond with a specific SIP message (e.g. a SIP 380 "Alternative Service" response) indicating that the call is an emergency. Upon receiving this indication, the UE establishes an Emergency PDN connection (using the Emergency APN) and performs an IMS emergency registration before re-initiating the call as an emergency session.

Option 2: IMS continues session internally as emergency session. In this approach, the network treats the call as an emergency without notifying the UE, thereby avoiding any extra round-trip signaling. The IMS core (P-CSCF) recognizes the dialed number as an emergency number and does not reject or redirect the call back to the UE. Instead, the P-CSCF interacts with the EPC network to establish an emergency call dedicated bearer for the ongoing session and inform E-CSCF this emergency session. This approach avoids the additional latency of a separate emergency attach procedure, thus significantly reducing the call setup time in the GEO satellite scenario.

6.X.3	Procedures

#### 6.x.3.1	non-UE detectable IMS emergency session

#### Option 1：IMS inform the UE of the emergency session(Applicable to both roaming and non-roaming)

- The UE initiates a normal call session request message via NB-IoT NTN access.

- The P-CSCF in the IMS identifies the session as an emergency call based on the UE's called number and replies to the UE with a response (e.g. SIP 380) indicating an emergency call.

- Upon receiving the emergency indication from the network, the UE establishes an Emergency PDN connection to the EPC (using the Emergency APN) .

- After establish Emergency PDN connection, UE performs an IMS emergency registration.

- The UE resend INVITE request to initiates an emergency call session, the subsequent procedures are same to UE detectable IMS emergency session .

#### Option 2：IMS continue the session and establish dedicated-bearer for Emergency call(	Non-roaming scenario and roaming with Local Breakout)

- The UE sends a call request through a normal IMS signaling bearer, where the dialed number is an emergency call number (e.g., 110 or 911), without indicating it as an emergency call.

- Upon receiving the request, the P-CSCF recognizes the dialed number as an emergency number and interacts with the PCRF to establish an emergency call bearer.The emergency call bearer establishment follows the same process as described in  UE-detectable emergency call bearer setup procedure. After establishment, the P-CSCF forwards the INVITE message to the E-CSCF.

- Upon receiving the message, the E-CSCF interacts with the LRF via the MI interface to obtain the UE's location information. Based on this location information, it performs PSAP selection.

- The E-CSCF forwards the INVITE to the selected PSAP to complete the subsequent emergency call procedure.

- Editor’s note: For the solution of perform emergency call as normal call under the Non-UE detectable home-routed scenario is FFS.

### 6.1.3	Impacts to Services, Entities and Interfaces

P-CSCF:

-	Support not rejecting the request and continuing the emergency call setup p\rocedure when a UE initiates an emergency call by dialing an emergency number as normal call over NB-IoT (GEO) access in a Non-UE detectable emergency call scenario.

* * * End of change * * *