SA WG2 Meeting #171	S2-2509649

13 – 17 October 2025, Wuhan, China	(revision of S2-2509426 of 09375 of 08739)

Source:	LG Electronics, Nokia, Lenovo, NEC, Toyota, NTT DOCOMO, China Mobile, China Telecom, vivo, ZTE, Huawei, HiSilicon

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

Discussion

There are 8 solutions for KI#1 "Network energy related information exposure" TR 23.700-67v0.2.0.

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution # & title | 1 | 2 | 3 |
| 1: Enhancement of Exposure API to include UE categorization data | X |  |  |
| 2: Policy Control Considering Slice or 5GC Energy Consumption | X | X |  |
| 8: Estimation of Renewable Energy consumption | X |  |  |
| 9: Consumption energy source exposure | X |  |  |
| 10: Energy consumption calculation enhancements | X |  |  |
| 11: Energy Consumption estimation considering idle energy consumption | X |  |  |
| 12: Enhancement of energy related information exposure | X |  |  |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | X |  | X |

Proposal#1: It is proposed to discuss Sol#2 when handling interim agreements/conclusions for KI#2 because Sol#2 is considered more related to KI#2 "Support of service adjustments for the UE".

Proposal#2: It is proposed to discuss Sol#22 when handling interim agreements/conclusions for KI#3 because Sol#22 is considered more related to KI#3 "NF (re)selection and/or UP path adjustment based on energy related information".

Proposal#3: It is proposed to discuss other solutions than Sol#2 and Sol#22 related to KI#1 interim agreements.

Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1.Y	Agreed Principles for KI#1

The following principles are made based on the solutions in clause 6 and agreed for KI#1.

1)	The following additional energy related information is exposed by EIF:

A)	The exposure of additional information in association to the exposure of energy consumption information at a desired granularity, specifically,

a)	This additional information, if requested by the consumers of the exposure services of the EIF, is provided together with the Energy Consumption information at the desired granularity and is the result of postprocessing of the information the EIF already has collected (i.e. no new information is required beyond what the EIF already has collected from OAM and SMF).

b)	energy consumption per bit: this is defined as the calculated energy consumption of the desired granularity divided by the measured data volume of the granularity used to calculate the energy consumption.

c)	“Energy consumption category” that is a scalar with multiple values that are determined by the EIF using PLMN-operator defined rules (e.g. 5 categories of percentile) and the rules and values associated to each category are not in scope of 3GPP. This category information is applicable to the same scope (e.g. regional, PLMN-wide) as the scope of the information that the EIF is collecting for the same granularity for other UEs that are e.g. associated to ongoing exposure subscriptions. The scope of the information an EIF collects is implementation and deployment specific.

B)	Per UE application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session with the following restriction:

a)	The application ranking related information exposed to the AF is only about the applications that the AF has permission/authorization with the applications and for which has requested the ranking. This permission/authorization is performed by the NEF. The EIF performs the ranking only of the ongoing applications subject to exposure.

NOTE 1:	The application ranking related information is exposed only to AF(s) related to those applications.

NOTE 2:	It is assumed that a single EIF is involved for all the applications that can be exposed together as the application ranking related information.

2)	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF depending on SA5 feedback.

3)	New formulas taking idle energy consumption into account will be discussed in SA2#172.

* * * * End of Changes * * * *