3GPP TSG-WG SA2 Meeting #171	S2-2509734

13th – 17th October, 2025, Wuhan, China	(revision of S2-2509708, 9507, 8835)

Source:	Lenovo (pen holder)

Title:	[WT#1.2, Slicing] Proposal to study Network Slicing in 6G

Document for:	Discussions and Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides a summary of the submitted papers on WT#1.2 on Network Slicing in 6G and is to be used for the way forward.

# 1. Discussion

## 1.1	Summary of input documents

This document merges the content of the proposals submitted to SA2#171 meeting on the WT on Network Slicing in 6G.

It is assumed that firstly agreement on the WT scope is achieved and afterwards agreement on the KI(s) description.

All considered papers are listed in the table:

| TD# | Network Slicing in 6G | Source |
| --- | --- | --- |
| S2-2508300 | 23.801-01: [WT#1.2, Slicing] Support for Network Slicing | OPPO |
| S2-2508352 | 23.801-01: [WT#1.2, Network Slicing] Network Slicing | vivo |
| S2-2508380 | 23.801-01: [WT#1.2, Network Slicing] Support of Network Slicing in 6G | Samsung |
| S2-2508551 | 23.801-01: [WT#1.2, Slicing] Scope and New Key Issue for WT#1.2 Slicing | LG Electronics |
| S2-2508571 | 23.801-01: WT and KI for Network Slicing in 6G | Huawei, HiSilicon |
| S2-2508683 | 23.801-01: [WT#1.2, Slicing] Key Issue and WT update for Slicing | Ericsson |
| S2-2508835 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G | Lenovo, NEC, Nokia |
| S2-2508902 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G | MediaTek Inc. |
| S2-2508931 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G. | InterDigital Inc. |
| S2-2508985 | 23.801-01: [WT#1.2, Slicing] Network Slicing Evolution in 6G | Apple |
| S2-2509094 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G. | NTT DOCOMO |
| S2-2509160 | 23.801-01: [WT#1.2, Slicing] Scope and key issue on Network Slicing in 6G | Ofinno |
| S2-2509163 | 23.801-01: [WT#1.2, Network Slicing] Work task and Key issue proposals on Network Slicing for 6G | Qualcomm Korea |
| S2-2509202 | 23.801-01: [WT#1.2, Slicing] Proposal to study Network Slicing in 6G. | [Lenovo, NEC, Nokia], Google |

The proposals of the documents is listed as follows:

[8835, Lenovo, NEC, Nokia]:

1.	Study the overall design and functionalities of the network slicing in 6G, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification.

.

NOTE 1:	Any Network Slicing-specific procedures enhancements (e.g. but not limited to between the UE and the network) that are stemming from the WT 1.2.x on Policy will be studied in this WT. Coordination between the WT on Slicing and the WT on Policy is expected.

NOTE 2:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and mandatory to support in the network.

[8551, LGE]: NOTE 2:	Potential privacy impact will be coordinated with SA3.

2.	Study impacts, if any, of the network slicing in 6G on the interworking and migration.

NOTE 3: Impacts to 6G RAN are possible.

[8300, Oppo]:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address areas of possible simplification comparing with 5G slicing design and how to simplify;

NOTE:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

2.	Study impacts, if any, of the 6G network slicing on the interworking with 5GS and potentially with EPS.

[8380, Samsung]:

1.	Study the overall design and functionalities of the 6G network slicing. The overall network slicing design will also consider:

a.	Identify and address any other areas of improvement for simplification (e.g., less options);

b.		Identify and address improvements of the application traffic mapping to network slice(s).

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[8571, Huawei]:

1.	Study the overall design and functionalities of the 6G network slicing. 5G network slicing is a start point for discussion. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification for both UE and network sides;

b.		Identify and address improvements of the application mapping to network slice(s).

NOTE: Coordination with WT#2 is needed on impacts, if any, of the 6G network slicing on the interworking and migration.

[8683, Ericsson]:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification;

b.		Identify and address improvements of the application traffic mapping to network slice(s).

NOTE 1:	It will be studied which of the 5G functionality (e.g. based on outcome of the 5G features support in 6G day-1 discussion) and potential new 6G functionality are mandatory to be supported in the UE and in the network.

NOTE 2:	Coordination with WT#1.2 policy is needed if the outcome of b) above has relations with policy.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

NOTE 3:	Coordination with RAN WGs is expected.

[8902, MediaTek]:

1.	Studying the overall design and functionalities of the network slicing in 6G. The overall network slicing design will also consider:

a.	Identification and addressing of any areas of possible improvement and simplification.

NOTE 1:	It will be studied which of the 5G functionality are to be supported and which are to be modified. On to be modified functionality, the study characterises the issues being addressed and the related solutions.

2.	Studying impacts, if any, of the  network slicing in 6G on the interworking and migration.

NOTE 2: Whether and how potentially support for WT3, WT4, WT5 and WT6 will be considered as part of the study.

[8931, InterDigital]:

1.	Study the overall design and functionalities of the 6G network slicing, which may include enhancing existing 5G functionality and developing new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification, including identifying and addressing any duplication of functionality.

b.		Identify and address improvements of the application traffic mapping to network slice(s).

c. 	Identify and address any areas that ease network slicing deployment.

NOTE:	It will be studied which of the enhanced and existing 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[8985, Apple]:

1.	Study the overall design and functionalities of the 6G network slicing. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification for slice management and slicing-related control plane signalling between UE and network. Study how to make network slicing more agnostic to UE behaviour;

b.		Identify and address improvements of the application traffic mapping to network slice(s) without impacting user privacy.

NOTE:	It will be studied which of the 5G slicing functionalities are essential, i.e. mandatory to be supported in the UE and/or in the network.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[9094, NTT DOCOMO]:

1.	Study the overall design and functionalities of the 6G network slicing, which should based on 5G slicing features and may include new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification, including device configuration;

b.		Identify and address improvements of the application traffic mapping to network slice(s), including enhancements to policy-based mechanisms (e.g., URSP) or a replacement of those mechanisms.

NOTE 1:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

NOTE 2: The study in this work task is related to and aligned with the WT1.2 Policy workstream.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[9160, Ofinno]:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification;

b.		Identify and address improvements of the application traffic mapping to network slice(s).

NOTE 1:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

NOTE 2:	This study will identify potential requirements impacting the design of the 6G policy framework (e.g., URSP design for 6G). The derived requirements will be provided to the WT#1.2 Policy scope for consideration.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[9163, Qualcomm]:

Taking network slicing for 5G system as defined in TS 23.501 [X] as a baseline, the scope of the network slicing work task includes the following potential enhancements:

1.	Identify and address improvements of the application traffic mapping to network slice(s).

2.	Study impacts, if any, of network slicing for 6G system on interworking and migration.

NOTE: Interworking and migration is studied in conjunction with WT#2.

[9202, Google]:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification, and reducing dependency on the UE to support network slicing.

b.		Identify and address improvements, including depreciation, of the 5G mechanisms for mapping application trafficto network slice(s).

NOTE:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

[8352, vivo]

## 5.X.	Key Issue #X: Support of Network Slicing

Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will study the following aspects:

-	Study how to simplify the network slicing, including:

-	Identify and address any areas of possible improvement and simplification

-	Study how to map the application traffic to the network slice, including:

-		Identify and address improvements of the application traffic mapping to network slice(s)

-	Study impacts, if any, of the 6G network slicing on the interworking and migration.

## 1.2	Open issues for discussion

- To bullet 1, there are the following directions:

  - To assume Network Slicing in 5G as baseline;

  - To be open to start from scratch a new design for network slicing in 6G. It means to avoid mentioning of any 5G or 6G in the bullet 1.

- Further open points to bullet 1:

  - Whether to include “new 6G functionality” in bullet 1.

  - Whether in bullet 1a to explicitly mention “and reducing dependency on the UE to support network slicing” as part of the simplification on the UE side.

- (Regarding bullet 1b) How to split the work between WT on Policy and WT on Slicing regarding the application traffic mapping to slices (and possible URSP impacts)?

  - Depending on the above resolution, whether to use “application mapping to network slice” or “application traffic mapping to network slice”?

- Whether to include “impacts to 6G RAN is possible” or “coordination with RAN WGs is expected”.

# 2. Proposal for the TR

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.2 Scope on Network Slicing

The scope of the network slicing work task includes:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point including the following:

a.	Identify and address any areas of possible improvement and simplification.

b.		Identify and address any improvements of the application traffic mapping to network slice(s)/data session(s).

NOTE 1:	Coordination between this WT and the WT 1.2.x on Policy is expected.

NOTE 2:	It will be studied which of the network slicing functionalities are mandatory to support in the UE and mandatory to support in the network.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 3:	Impacts to RAN are possible and coordination with RAN WGs is expected.

**** End of Changes ****