SA WG2 Meeting #S2-172	S2-2511020

17 - 21 November, 2025, Dallas, USA	(revision of 11002, 10388)

Source:	vivo, Samsung, Xiaomi, Futurewei

Title:	Conclusions on KI#2: Authorization and Revocation to Support Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Add new conclusion for KI#2.

1. Introduction

The contribution proposes to add new conclusion for key issue#2 based on interim agreement in clause 7.1.2.

7.1.2 Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service The following principles are agreed for KI#2: a) The Sensing Service authorization consists of two steps: 1) Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9]; 2) Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality; NOTE: In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway and the wording may need alignment according to the conclusion of key issue 1. b) Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase. Table 7.1.2-1: Sensing Authorization information for Sensing Service AF Authorization Data; Description. AF ID; Identifier used to identify the AF.. Allowed/Not allowed area for sensing; Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1).. Allowed/Not allowed time period for sensing; Indicate the allowed time period within which AF can trigger a particular sensing service operation.. (Allowed) sensing service type; Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations.. NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed.. c) The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorisation of the sensing service request. d) Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. e) The AF can cancel the on-going Sensing Service Request when needed. f) There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

Merge of KI#2 conclusions:

As per the guidance from the rapporteur, the revision of this document also captures all or part of the proposals of the following documents:

Table 1-1: List of merged documents of KI#2.

| Tdoc#=13 | Subject | Source | Clause(s) |
| --- | --- | --- | --- |
| S2-2510388 | 23.700-14: Conclusions on KI#2: Authorization and Revocation to Support Sensing Service | vivo | Clause 8 |
| S2-2509909 | 23.700-14: KI#2: Conclusions and Interim Agreements | LG Electronics | Clause 8 |
| S2-2509954 | 23.700-14: Proposal on Interim agreements update for KI#2 | OPPO | Clauses 7 & 8 |
| S2-2509990 | 23.700-14: Proposed Conclusions for KI#2 | CEWiT | Clause 8 |
| S2-2510005 | 23.700-14: KI#2 Conclusion | Apple | Clauses 7 & 8 |
| S2-2510036 | 23.700-14: FS_Sensing_ARC -- update on Interim agreements for KI#2 | China Mobile Com. Corporation | Clause 7 |
| S2-2510069 | 23.700-14: Proposal for KI#2 conclusion | InterDigital Communications | Clauses 7 & 8 |
| S2-2510196 | 23.700-14: Conclusion proposal to KI#2 Authorization and Revocation to Support Sensing Service | ZTE | Clauses 7 & 8 |
| S2-2510410 | 23.700-14: KI#2: conclusion update proposal | Huawei | Clauses 7 & 8 |
| S2-2510483 | 23.700-14: Interim agreement update for FS_Sensing_ARC KI#2 | Lenovo | Clause 7 |
| S2-2510678 | 23.700-14: Interim agreement update and final conclusion for KI#2 | Samsung Guangzhou Mobile R&D | Clauses 7 & 8 |
| S2-2510689 | 23.700-14: Conclusion for KI#2 Authorization and Revocation to Support Sensing Service | Beijing Xiaomi Electronics | Clause 8 |
| S2-2510761 | 23.700-14: FS_Sensing_ARC: Additional principles for KI#2 on Authorization and Revocation to support sensing service | Qualcomm Incorporated | Clause 7 |

Observations (by pen-holder):

Based on pCRs in Table 1-1, pen-holder has following observations:

- Observation#1: 3 pCRs touched only clause 7 "Interim agreements", while all other 10 pCRs either touched both clause 7 "Interim agreements" and clause 8 "Conclusions" or only clause 8 "Conclusions".

- Observation#2: 4 pCRs proposed to change "Sensing authorization functionality" to "Sensing Function (SF)", 1 pCR proposed to change to "Sensing Control Function (SCF) ", 1 pCR proposed to change to "Sensing Gateway Function", and 1 pCR proposed to change to "Sensing authorization function".

- Observation#3: only 1 pCR proposed to capture the procedures for sensing authorization and sensing revocation and all other pCRs do not touch procedures aspects in the TR conclusion.

Outstanding issues to be resolved:

- Whether SF further verifies the exposed sensing result/data after the successful sensing service request authorization. (S2-2509954)

- Whether assignment of a sensing operation ID for identifying the sensing operation and the sensing result associated to the authorized sensing service request after successful authorization of sensing service request is in the scope of KI#2. (S2-2510069)

Proposals (by pen-holder):

Based on above observations, pen-holder has following proposals for the way forward:

- Proposal#1: To copy the approved text in clause 7.1.2 to new clause 8 as conclusion baseline, based which to capture the required updates and new changes, while keeping clause 7.1.2 untouched.

- Proposal#2: Change "Sensing authorization functionality" to "Sensing Function (SF)".

- Proposal#3: The procedures for sensing authorization and sensing revocation are not captured in the TR conclusion, i.e. can be captured in the normative work.

Merge notes (by pen-holder):

- Some proposed changes of a pCR are not merged due to they are covered by other proposed changes of another pCR.

- Changes related to above outstanding issues are not merged yet.

- Same or overlapped changes from different pCRs are only merged once.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

8.X Conclusions for KI#2 Authorization and Revocation to Support Sensing Service

To resolve the KI#2 Authorization and Revocation to Support Sensing Service, the following conclusions are adopted for normative work:

a)	The Sensing Service authorization consists of two steps:

1)	Authorization of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	The Sensing Function (SF) performs authorization of the AF's Sensing Service Request based on the sensing authorization information.

NOTE 1:	In case of the SF contains Sensing control functionality (SCF) and Sensing Processing functionality (SPF), the SCF performs the authorisation and revocation of Sensing Service Request.

b)	The SF authorizes the Sensing Service Request from the AF, considering the example of Sensing authorization information described in Table 8.2-1, and

1)	proceeds the Sensing Service Request based on requested sensing requirements, if Sensing Service Request from the AF is authorized.

2)	rejects the Sensing Service request, if Sensing Service Request from the AF is not authorized.

NOTE 2:	Whether and how to perform partial authorization of Sensing Service Request will be determined during the normative phase.

NOTE 3:	Whether and how to pause the ongoing sensing service operation will be determined during the normative phase.

Table 8.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated the AF to request the sensing services operations (NOTE 1). |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which the AF can be authorized as a Sensing Service Consumer. |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

NOTE 4:	The further detail of the sensing authorization information will be determined during the normative phase.

c)	The Sensing Authorization information is pre-configured or configured by OAM, and stored in the SF for authorization of the sensing service request.

d)	The SF can revoke the previously authorized Sensing Service Request when the authorization criteria are no longer met.

e)	The AF can cancel the on-going Sensing Service Request when required. Then, the SF may terminate the on-going sensing service operation between the SF and the SE upon cancellation.

f)	There is no need for authorization for the gNB as the Sensing Entity, as the gNB is deployed by operator.

# * * * * End of changes * * * *