3GPP TSG-SA3 Meeting #123	S3-252533

Goteborg, Sweden, 25 – 29 August 2025

# Source:	Interdigital

# Title:	New Key Issue for Security and Privacy for UE Data Collection Transfer

# Document for:	Endorsement

Agenda Item:	6.1.7

# 1	Decision/action requested

It is proposed to endorse this new Key Issue for Security and Privacy for UE Data Collection Transfer.

# 2	References

[1]	Security for AI/ML Phase 2 living document

# 3	Rationale

This contribution proposes a new key issue on security and privacy for UE data collection transfer used for UE-side model training.

# 4	Detailed proposal

It is proposed to approve the following key issue to be included in the living document [1].

* * * * Start of Changes (all text new) * * * *

## 5.X	Key issue #X: Security and Privacy for UE Data Collection Transfer

### 5.X.1	Key issue details

As part of the UE data collection process, the UE uses standardized transfer of standardized data over UP, where the MNO has full visibility for standardized data (as per clause 4.2 of TR 23.700-04 [x]).

The standardized data reported by the UE over UP may include privacy sensitive data, vendor proprietary information that if leaked may expose the MNO to liabilities. The measurements as data reported if tampered with, may impact negatively the UE side model training and the resulting associated AI model performance.

### 5.X.2	Security threats

Lack of security during the establishment of the UP-communication path between the UE and the Core Network may expose the UE and network to impersonation attacks (e.g., unauthorized service access).

Lack of confidentiality protection of the data transfer from the UE may expose privacy sensitive (e.g., UE or session identifier) or vendor proprietary data (e.g., device specific information) to an eavesdropper.

Lack of integrity protection of the data transfer from the UE may expose the system to data poisoning attacks potentially leading to poor AI model performance and negative impact to NR air interface operations.

### 5.X.3	Potential security requirements

The system shall support secure means to establish UP communication path between the UE and the 5GC for the transfer of UE side model training data.

Confidentiality protection, Integrity protection and replay protection shall be supported for the communication between the UE and the 5GC used for the transfer of UE side model training data.

.

The 5G System should provide means for mitigating trackability and linkability attacks on the UE during communications with 5GC for the transfer of the UE side model training data.