3GPP TSG-SA3 Meeting #123	S3-252534

Goteborg, Sweden, 25 – 29 August 2025

# Source:	Interdigital

# Title:	New Key Issue for Support of PSK Mode between the UE and UPF

# Document for:	Endorsement

Agenda Item:	6.1.6

# 1	Decision/action requested

It is proposed to endorse this new Key Issue for Support of PSK Mode between the UE and UPF.

# 2	References

[1]	Security for QUIC/TLS living document

# 3	Rationale

This contribution proposes a new key issue for the support of PSK mode between the UE and UPF.

# 4	Detailed proposal

It is proposed to approve the following key issue to be included in the living document [1].

* * * * Start of Changes (all text new) * * * *

## 5.X	Key issue #X: Support of PSK Mode between the UE and UPF

### 5.X.1	Key issue details

This key issue aims to study how to support PSK based security (e.g., used in MP/QUIC) between the UE and UPF. Compared to certificate-based authentication, PSK mode presents advantages in terms of simplified credentials management, communication establishment performance (e.g., 0-RTT).

### 5.X.2	Security threats

Without secure means to properly setup, revoke or update the PSK the system may be exposed to various security vulnerabilities. For example, a compromised PSK on the UE side may be used by an attacker to gain unauthorized access to the UPF.  Similarly, the lack of proper re-keying mechanism may be exploited by an attacker in possession of a leaked PSK to acquire past session data.

### 5.X.3	Potential security requirements

The system should support secure means for lifecycle management (setup, update, revoke) of PSK used between the UE and UPF.