3GPP TSG-SA3 Meeting #123	S3-252584

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0216 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct test case on GTP filtering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MITRE-FFRDC, Deutsche Telekom, BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For steps 4 and 7, the NP shall not send a response back to the tester because the message is supposed to be discarded. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct TC to indicate no response is sent back to tester after the message is discarded. Rather than “any” response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tester may assume a response from network product is expected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6.2.3, 4.2.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** Start of 1st Change**************

##### 4.2.6.2.3	GTP-C Filtering

Requirement Name: GTP-C Filtering

Requirement Reference: In accordance with industry best practice

Requirement Description:

The following capability is conditionally required:

-	For each message of a GTP-C-based protocol, it shall be possible to check whether the sender of this message is authorized to send a message pertaining to this protocol.

NOTE 1: 	The check could be performed e.g. against an allow list or disallow list of permitted message type / sender identity combinations.

-	At least the following actions should be supported when the check is satisfied:

-	Discard: the matching message is discarded.

-	Accept: the matching message is accepted.

-	Account: the matching message is accounted for, i.e. a counter for the rule is incremented. This action can be combined with the previous ones. This feature is useful to monitor traffic before its blocking.

This requirement is conditional in the following sense: It is required that at least one of the following two statements holds:

-	The Network Product supports the capability described above and this is stated in the product documentation.

-	The Network Product's product documentation states that the capability is not supported and that the Network Product needs to be deployed together with a separate entity which provides the capability described above.

NOTE 2: 	Such a separate entity could e.g. be a GTP Firewall.

NOTE 3: 	Test cases for this separate entity are not provided in the present document, but are believed to be similar to them.

NOTE 4: The test cases are only applicable to all network product classes utilizing GTP-C based protocol.

Threat References: TR 33.926 [4], clause 5.3.7, Denial of service

Test case:

The test case described here apply only when GTP-C filtering is provided on the Network Product itself.

Test Name: TC_GTP-C_FILTERING

Purpose:

To verify that the network product provides filtering functionalities for incoming GTP-C messages. In particular this test case verifies that:

1.	The network product provides filtering of incoming GTP-C messages on any interface.

2.	It is possible to block all GTP-C messages on those network product interfaces where they are unwanted.

3.	It is possible to specify defined actions for each rule.

Procedure and execution steps:

Pre-Conditions:

-	The network product has at least two physical interfaces, named if1 and if2.

-	The tester has the privileges to configure GTP-C filtering on the network product.

-	The vendor declares that the GTP-C filtering is supported.

-	The vendor includes a guideline to configure the GTP-C filtering in the documentation accompanying the network product.

-	A network traffic generator or a pcap file containing the GTP-C messages is available.

-	A network traffic analyser on the network product (e.g. tcpdump) is available.

Execution Steps

1.	The tester log in the network product.

2.	The tester configures the network product with the following rules:

a)	Accept only GTP-C EchoRequest messages on if1.

b)	Discard all GTP-C messages on if2.

c)	For each rule above the accounting is also enabled.

3.	The tester turns on the network traffic analyser on if2.

4.	The tester sends on if2 EchoRequest messages replaying a pcap file or using a network generator.

a)	Using the network analyser the tester verifies that the network product correctly receives the EchoRequest messages on if2.

b)	Using the accounting, the tester verifies that the messages are discarded and that no response is sent back by the network product.

5.	The tester sends to if1 EchoRequest messages replaying a pcap file or using a network generator.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-C EchoRequest messages are not discarded because EchoResponse messages are sent back by the network product.

6.	The tester verifies that the matching messages are correctly accounted for both rules.

7.	The tester sends to if1 GTP-C messages different from EchoRequest replaying a pcap file or using a network generator.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	Using the accounting, the tester verifies that the messages are discarded and that no response is sent back by the network product.

8.	The tester deletes the previous rules and configures a new rule, i.e. to accept only GTP-C EchoRequest on if1 coming from a certain IP Address named IP1.

9.	The tester sends GTP-C EchoRequest messages with source IP Address set to IP1:

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-C EchoRequest messages are not discarded and EchoResponse messages are sent back by the network product.

10.	The tester sends GTP-C EchoRequest messages with source IP Address set to IP2 different from IP1 using a network traffic generator or replaying a pcap file.

a)	Using the network analyser the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-C EchoRequest messages are discarded and that no EchoResponse messages are sent back.

Expected Results:

-	For steps 4, 5, 6 and 7 the tester receives GTP-C EchoResponse messages from if1 only.

-	For steps 4, 5, 6 and 7 the messages matching the rules are correctly accounted.

-	For steps 8, 9, 10 the tester receives GTP-C EchoResponse messages only for the authorized source IP address.

Expected format of evidence:

-	The used tool(s) name and version information

-	Settings and configurations used

-	Pcap trace

-	Screenshot

#### 4.2.6.2.4	GTP-U Filtering

Requirement Name: GTP-U Filtering

Requirement Reference: In accordance with industry best practice

Requirement Description:

The following capability is conditionally required:

-	For each message of a GTP-U-based protocol, it shall be possible to check whether the sender of this message is authorized to send a message pertaining to this protocol.

NOTE 1: 	The check could be performed e.g. against an allow list or disallow list of permitted message type / sender identity combinations.

-	At least the following actions should be supported when the check is satisfied:

-	Discard: the matching message is discarded.

-	Accept: the matching message is accepted.

-	Account: the matching message is accounted for, i.e. a counter for the rule is incremented. This action can be combined with the previous ones. This feature is useful to monitor traffic before its blocking.

This requirement is conditional in the following sense: It is required that at least one of the following two statements holds:

-	The Network Product supports the capability described above and this is stated in the product documentation.

-	The Network Product's product documentation states that the capability is not supported and that the Network Product needs to be deployed together with a separate entity which provides the capability described above.

NOTE 2:	Such a separate entity could e.g. be a GTP Firewall.

NOTE 3:	Test cases for this separate entity are not provided in the present document, but are believed to be similar to them.

NOTE 4:	The test cases are only applicable to all network product classes utilizing GTP-U based protocol.

Threat References: TR 33.926 [4] clause 5.3.7, Denial of service

Test case:

The test case described here apply only when GTP-U filtering is provided on the Network Product itself.

Test Name: TC_GTP-U_FILTERING

Purpose:

To verify that the network product provides filtering functionalities for incoming GTP-U messages. In particular this test case verifies that:

1.	The network product provides filtering of incoming GTP-U messages on any interface.

2.	It is possible to block all GTP-U messages on those network product interfaces where they are unwanted.

3.	It is possible to specify defined actions for each rule.

Procedure and execution steps:

Pre-Conditions:

-	The network product has at least one physical interface named if1 and may have another physical interface named if2.

-	The tester has the privileges to configure GTP-U filtering on the network product.

-	The vendor declares that the GTP-U filtering is supported.

-	The vendor includes a guideline to configure the GTP-U filtering in the documentation accompanying the network product.

-	A network traffic generator or a pcap file containing the GTP-U messages is available.

-	A network traffic analyser on the network product (e.g. tcpdump) is available.

NOTE:	If the network product has only one physical interface named if1, execution steps on if2 are not needed.

Execution Steps

1.	The tester log in the network product.

2.	The tester configures the network product with the following rules:

a)	Accept only GTP-U EchoRequest messages on if1.

b)	Discard all GTP-U messages on if2.

c)	For each rule above the accounting is also enabled.

3.	The tester turns on the network traffic analyser on if2.

4.	The tester sends on if2 EchoRequest messages replaying a pcap file or using a network generator.

a)	Using the network analyser, the tester verifies that the network product correctly receives the EchoRequest messages on if2.

b)	Using the accounting, the tester verifies that the messages are discarded and that no response is sent back by the network product.

5.	The tester sends to if1 EchoRequest messages replaying a pcap file or using a network generator.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-U EchoRequest messages are not discarded because EchoResponse messages are sent back by the network product.

6.	The tester verifies that the matching messages are correctly accounted for both rules.

7.	The tester sends to if1 GTP-U messages different from EchoRequest replaying a pcap file or using a network generator.

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	Using the accounting, the tester verifies that the messages are discarded and that no response is sent back by the network product.

8.	The tester deletes the previous rules and configures a new rule, i.e. to accept only GTP-U EchoRequest on if1 coming from a certain IP Address named IP1.

9.	The tester sends GTP-U EchoRequest messages with source IP Address set to IP1:

a)	Using the network analyser, the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-U EchoRequest messages are not discarded and EchoResponse messages are sent back by the network product.

10.	The tester sends GTP-U EchoRequest messages with source IP Address set to IP2 different from IP1 using a network traffic generator or replaying a pcap file.

a)	Using the network analyser the tester verifies that the messages are correctly received by the network product.

b)	The tester verifies that the GTP-U EchoRequest messages are discarded and that no EchoResponse messages are sent back.

Expected Results:

-	For steps 4, 5, 6 and 7 the tester receives GTP-U EchoResponse messages from if1 only.

-	For steps 4, 5, 6 and 7 the messages matching the rules are correctly accounted.

-	For steps 8, 9, 10 the tester receives GTP-U EchoResponse messages only for the authorized source IP address.

Expected format of evidence:

-	The used tool(s) name and version information

-	Settings and configurations used

-	Pcap trace

-	Screenshot

************** End of 1st Change**************