3GPP TSG-SA3 Meeting #123	S3-252588

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0089 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TC on gNB replay protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MITRE-FFRDC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For UP replay protection tester must at least verify gNB does not forward UP packets on the N3 interface. Replayed RRC packets may result in either further RRC procedures in N2 interface (NAS registration) and/or in response to UE on air interface (RRC setup, RRC connection Reconfiguration). Therefore both of these should be verified. Tester shall ensure no further RRC messages are exchanged due to the replayed RRC packet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The gNB should have an established PDU and RRC connection prior to the start of the tests. Changed the RRC replay TC to require two replay protection verifications: - Verify no user plane or RRC packets are forwarded on N2 interface. - Verify response message is not sent on NG RAN air interface Changed the user plane replay TC to require one replay protection verifcation: - Verify no user plane or RRC packets are forwarded on N3 interface. For UP replay, gNB may not send response to UE, therefore, this becomes an optional check. Verifying gNB logs is an optional check in both TC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test does not provide sufficient evidence of replay protection in the gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.8, 4.2.2.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***********Start of Change ***********

##### 4.2.2.1.8	Replay protection of user data between the UE and the gNB

Requirement Name: Replay protection of user data between the UE and the gNB.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of user data as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.4 – User plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-REPLAY_gNB

Purpose:

To verify that the user data packets are replay protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface.

-	The tester shall have access to the N3 interface.

-	The gNB and UE shall have established an PDU session.

-	The tester shall activate the user plane integrity protection of user data packets.

Execution Steps:

1.	The tester shall capture the user plane data sent between UE and gNB using any network analyser over the NG RAN air interface.

2.	Tester shall filter user plane data packets sent between UE and gNB.

3.	Tester shall replay the captured user plane packets or shall use any packet crafting tool to create a user plane packet identical to the captured user plane packet and replay to the gNB.

4.	Tester shall check whether the replayed user plane packets were processed by capturing traffic over N3 interface to see if any of the replayed user plane packets have been forwarded by the gNB. Additionally, tester may verify the gNB log files to check for entries about UP packet discard and/or tester may capture traffic over NG RAN air interface to see whether any corresponding response message is sent by the gNB.

5.	Tester shall confirm that gNB provides replay protection by dropping/ignoring the replayed packet if no replayed user plane packets have been forwarded over the N3 interface. Tester may also confirm that no corresponding response message is sent by the gNB to the replayed packet and that the gNB logs include corresponding log entries indicating integrity check failed or replay detected.

6.	Tester shall verify from the result that if the replayed user plane packets are not accepted by gNB, the NG RAN air interface is replay protected.

Expected Results:

The user plane packets sent between the UE and gNB over the NG air interface are replay protected.

Expected format of evidence:

-	Evidence suitable for the interface, e.g. Screenshot containing the operational results.

-	Log files, e.g., containing corresponding log events.

##### 4.2.2.1.9	Replay protection of RRC-signalling

Requirement Name: Replay protection of RRC-signalling.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of RRC-signalling as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-RRC-REPLAY_gNB

Purpose:

To verify the replay protection of RRC-signalling between UE and gNB over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments.

-	Tester shall have knowledge of the integrity algorithm and the corresponding protection keys.

-	The tester shall have access to the NG RANs air interface.

-	The tester shall have access to the N2 interface.

-	The gNB and UE shall have established an RRC connection (or are ready to start exchanging RRC messages).

-	The tester shall activate the integrity protection of RRC-signalling.

Execution Steps:

1.	The tester shall capture the data sent between UE and the gNB using any network analyser over the NG RAN air interface.

2.	Tester shall filter RRC signalling packets.

3.	Tester shall check for the PDCP COUNT of the filtered RRC signalling packets and shall use any packet crafting tool to create RRC signalling packets identical to the captured packets or the tester shall replay the captured RRC uplink packet to the gNB to perform the replay attack over gNB.

4.	Tester shall check whether the replayed RRC signalling packets were processed by the gNB or not, by capturing over NG RAN air interface to see if any corresponding response message is received from the gNB.  Additionally, tester shall capture the traffic on the N2 interface to see if any of the replayed RRC signalling packets trigger further RRC message exchange(s). The tester may also check the gNB log files for entries about RRC packet discard.

5.	Tester shall confirm that gNB provides replay protection by dropping/ignoring the replayed packet if no corresponding response is sent by the gNB to the replayed packet.  Additionally, tester shall ensure no further RRC procedures are triggered by replayed RRC signalling packets. The tester may confirm the gNB logs include corresponding log entries indicating integrity check failed or replay detected. The tester may need to cross check verification methods to confirm the gNB provides full replay protection )

NOTE 1: Any one these methods alone may not provide sufficient evidence to confirm that gNB provides full replay protection. For example, the lack of corresponding response message sent by the gNB over the NG RAN air interface by itself cannot confirm that gNB provides replay protection because some signalling packets may not result in corresponding response messages (e.g., RRC Reconfiguration Complete message).

Expected Results:

The RRC signalling over the NG RAN air interface is replay protected.

The tester shall ensure that no new RRC messages are sent in response to the replayed packets and that the gNB does not update or change UE state based on the replayed packet.

Expected format of evidence:

-	Evidence suitable for the interface, e.g. Screenshot containing the operational results.

-	Log files, e.g., containing corresponding log events.

***********End of Change ***********