3GPP TSG-SA3 Meeting #123	S3-252605

# Goteborg, Sweden, 25 – 29 August 2025

Source:	ZTE

Title:	Resolving ENs for group inventory

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

To protect privacy of AIoT device identifiers, temporary IDs are recommended during inventory and command messages. During last meeting, several options have been endorsed in S3-252326 with ENs left to be resolved.

So, this contribution is made to address the following ENs in group inventory (clause 4.1 in S3-252326):

Editor’s Note: whether AIOTF or ADM computes and checks T-ID is FFS.

Editor’s Note: in case AIOTF computes T-ID, how the KAIoTF is derived in ADM and how AIOTF retrieves the KAIoTF is FFS. In case ADM computes T-ID, the impact of interaction between AIOTF and ADM and the analysis of load of ADM is FFS.

Editor’s Note: whether AIOTF can acquire device ID from ADM is FFS.

The above ENs are related to where the temp ID is calculated and corresponding procedures. Considering the initial ideas on AIOTF functions and the load of the ADM, we propose to derive and check the temporary ID in the AIOTF. In this way, the Kaiotf needs to be calculated in the ADM and transferred to the AIOTF. In addition, the AIOTF needs to obtain the device ID from the ADM in group inventory scenario.

Editor’s Note: whether and how to address Replay attack (e.g., replay RANDAIOT_n to track the same T-ID) or DoS attack (e.g., energy depletion in AIoT device) is FFS.

Editor’s Note: whether a random number from device is required is FFS

In the endorsed solution, RANDaiot_n is included in the paging message, which may cause replay attack and DoS attack. Therefore, we propose to modify the solution to pre-configure the initial temporary ID, so that the random number does not need to be carried in the paging message, also reducing the impact to paging message. In this way, a random number is needed from device to update the temp ID.

Editor’s Note: which input key (e.g., KAIoT or KAIoTF) to be used is FFS

Editor’s Note: which algorithm will be used for temp ID generation is FFS.

To solve the above ENs, we provide a method of using HMAC and Kaiotf.

* * * First Change * * * *

4.1	Procedure for AIoT Device identifier protection during group inventory

The content below is from S3-252327 [1]. It includes one privacy option for the Group inventory.

Option for Group Inventory Procedure based on filtering information

Figure 4.1.x-1:  Privacy protection of AIoT device ID in inventory with filtering information

- The AIoT Device has a pre-configured initial Temp ID, the Network (the ADM) also have the initial Temp ID stored together with the device long term key, KAIOT in the AIoT Device profile.

1.	Step 1-6 of clause 6.2.2 (Procedure for Inventory) or clause 6.2.3 (Procedure for command) of TS 23.369 [2] is performed. The AIoT_Inventory request or the AIoT_Command_request contains the filtering information.

Editor’s Note: issues related filtering information is FFS.

2.	The AIOTF sends AIoT_DeviceID Request to ADM to fetch the device IDs and current Temp IDs (i.e. T-IDn list) of multiple AIoT devices identified by filtering information. Filtering information is included in the request.

Base on local policy, AIOTF determines whether or not to fetch Kaiotf from ADM. If an AIOTF key is needed, an AIOT key indication is included in the message. Kaiotf is derived from Kaiot in the ADM as, Kaiotf = HMAC (TIDn), using Kaiot as the key.

- The ADM sends AIoT_DeviceID Response to the AIOTF.

- The AIoTF shall include the filtering information in an Inventory Request and send the Inventory Request to the NG-RAN.

- The NG-RAN shall send a Paging message containing the filtering information.

- Upon receiving the Inventory request, the AIoT device checks if the filtering information matches with the configured filtering information. If they match, the AIoT device generates a new RANDd, constructs T-IDn+1, and upates T-IDn to T-IDn+1. T-IDn+1 is generated as specified in clause B.D.

If the filtering information does not match, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

Editor’s Note: the format of the configured filtering information is FFS.

- The AIoT device shall include the T-IDn and RANDd in an D2R message and send it to the NG-RAN.

- The NG-RAN shall send an Inventory Report containing the T-IDn and RANDd to the AIOTF.

- Upon receiving the Inventory response, the AIOTF shall find a matching T-IDn. If there is a match, the AIOTF constructs T-IDn+1, and upates T-IDn to T-IDn+1. T-IDn+1 is generated in the same way as AIoT device did in step 6.

- AIOTF sends an AIoTID Update Request to ADM. Both Device ID and T-IDn+1 are included in the message.

- After receiving the Temp ID update request, the ADM upates T-IDn to T-IDn+1in the AIoT Device profile.

- The ADM sends AIoT_T-ID Update Response to the AIOTF.

- The AIOTF shall construct an AIoT_Inventory Notify including the AIoT device ID that corresponds to the received T-ID.

- The AIOTF continues with the procedures as specified in clause 6.2.2 or clause 6.2.3 of TS 23.369 [2].

Editor’s Note: Whether network can provide network assigned T-ID to the AIoT device in command procedure is FFS.

B.D	Temporary ID generation scheme

Temporary ID s are generated based the following parameters:

-	AIoT Temp ID;

-	RANDd;

HMAC is used as the temp ID generation algorithm and Kaiotf is used as the key.

* * * End of Changes * * * *