3GPP TSG-WG3 Meeting #123	S3-252639

Goteborg, Sweden, 25 – 29 August 2025	(revision of S3-251904)

Source:	KDDI, Boost Mobile Network

Title:	Discussion on Security Aspects for IMS resiliency study

Document for:	Discussion

Agenda Item:	6.1.2

Work Item / Release:	T.B.D / Rel-20

Abstract: This paper provides background for Security Aspects for IMS resiliency study.

1. Introduction

The contribution provides background for Security Aspects for IMS resiliency study.

2.Background

At the SA#107 and CT#107 meetings, a draft IMS resiliency study item and discussion paper (SP-250245 [1]/SP-250246 [2]/CP-250210 [3]/CP-250243 [4]) were discussed, which addressed the following work tasks:

-	WT#1: Study whether and how the UDM provides single authentication between UEs and the 5GC and the IMS during the registration procedure.

-	WT2: Study whether and how 5GC notify the IMS status indicating failure or recovery conditions to UEs to avoid signalling storms and to mitigate IMS failures.

-	WT3: Study whether and how the 5GC in coordination with the UE configure the signalling frequency and timing of signalling based on the operator’s policy to mitigate negative impacts on the 5GC and/or the IMS.

Based on the discussion, SA #107 concluded the guidance and recommendation as follows:

At SA #107 (SP-250418 [5])

- This topic is added to the list of potential Rel-20 5GA topics (See SP-250246 for reference)

- Based on CT WGs feedback, moderated discussion in SA2 can start after SA2#168 (April) meeting, if needed

At CT4#128 meeting, the LS (C4-251492 [6]) on feedback for IMS resiliency to SA2 (CCed SA3 and CT1) was agreed. The feedback of WT#1 contains as follows:

- Feedback on Work Task #1: This WT cannot initially be studied in CT4, since it is under the limit of SA3.

At SA3#129 meeting, new SID for Security Aspects for IMS resiliency study (S3-252352) was endorsed.

Based on the discussions above, the proponent companies propose a new SID on Security Aspects for IMS Resiliency.

3. Discussion

Most MNOs apply the IMS framework for their voice services and will still need to continue operating IMS even in the 6G era. However, large-scale communication failures related to or affected by IMS have occasionally occurred on not a few operators’ mobile networks in the past few years. It goes without saying that strengthening the resiliency of IMS is necessary as MNOs continue to operate it over the coming years.

When a UE subscribes to IMS service, it sends a USIM authentication request to 5GC first, then to the IMS second. This means the UDM must process same authentication twice. When I-CSCF sends Cx-Query/Cx-Select-Pull request to the UDM, the UDM checks whether the UE is allowed to register in the P-CSCF if the UE is registered already with the network, and sends Cx-Query/Cx-Select-Pull response with the S-CSCF name as defined in TS 23.228 [7]. This means the procedure is executed in duplicate, causing extra signaling.

The procedures for IMS AKA (authentication and key agreement) are defined in clause 6.1 of TS 33.203 [8]. When the UE initiates IMS registration, the S-CSCF retrieves authentication information such as RAND, XRES, CK and IK from the UDM and performs the IMS AKA procedures with the I/P-CSCF and the UE. However, in the non-IP Sec environment, mutual authentication using CK and IK is not necessary.

Figure 1 shows signaling flows of the IMS AKA for an unregistered IM subscriber and successful mutual authentication with no synchronization error derived from Figure 4 of TS 33.203 [8] with the authentication information added.

Figure 1: The IMS AKA for an unregistered IM subscriber and successful mutual authentication with no synchronization error in the sequence flow description derived from Figure 4 of TS 33.203 [7]

Observation 1: UDM executes excessive authentications twice.

Even if the service is being provided normally, when congestion occurs and the response from IMS/5GC becomes slow, massive UEs attempts to register to the network in cycles, increasing the load on the IMS/5GC and leading to a cascade of failures. In the registration procedure for IMS AKA, which triggers failures, eight processing steps (i.e., step 3-4, step 6-7, step 13-14 and step 17-18 in the Figure 1) are required in the UDM. When there is a large amount of registration requests from UEs, the UDM congestion occurs, making it easier for failures to spread not only to IMS but also to 5GC. In large-scale failures, signals from millions of subscribers may flow into the UDM. By reducing the number of processing steps, the load on the UDM can be significantly decreased, thereby helping to prevent congestion and ensuring smoother operation.

Observation 2: Excessive amount of signaling may cause congestion.

Based on the lessons learned from the significant failure and aiming to achieve stable voice services with IMS, the proponent companies propose following WTs in a new SID on Security Aspects for IMS Resiliency.

WT1:	Identify non-resilient scenarios which are caused by IMS authentication, including:

Case 1: UDM executes excessive authentications twice.

Case 2: Excessive amount of signaling may cause congestion.

WT2:	Study the potential solution to address the identified scenarios.

Proposal: It is proposed that SA3 starts to study Security Aspects for IMS Resiliency.

4. Proposal

Source companies of this contribution propose following WTs in a new SID on Security Aspects for IMS Resiliency:

WT1:	Identify non-resilient scenarios which are caused by IMS authentication, including:

Case 1: UDM executes excessive authentications twice.

Case 2: Excessive amount of signaling may cause congestion.

WT2:	Study the potential solution to address the identified scenarios.

5. Reference

[1]		SP-250245: "Discussion on IMS resiliency study", KDDI, SK Telecom

[2]		SP-241972: "New WID on Study on IMS resiliency”, KDDI, AT&T, Deutsche Telekom, KPN N.V., SK Telecom, SoftBank, TOYOTA MOTOR CORPORATION, Rakuten Mobile

[3]		CP-250210: "Discussion on IMS resiliency study", KDDI, SK Telecom

[4]		CP-250243: "New WID on Study on IMS resiliency", KDDI, AT&T, Deutsche Telekom, Rakuten Mobile, SK Telecom, SoftBank, TOYOTA MOTOR CORPORATION

[5]		SP-250418: "SA Guidance to SA2 on Rel-20 5GA studies", SA2 Chair

[6]		C4-251492: "LS on feedback for IMS resiliency study", CT WG4

[7]		3GPP TS 23.228: "IP Multimedia Subsystem (IMS)"; Stage 2"

[8]		3GPP TS 33.203: "3G security; Access security for IP-based services"