3GPP TSG-SA3 Meeting #123	S3-252650

# Goteborg, Sweden, 25 – 28 August 2025

Source:	China Telecom, CableLabs, CAICT, ZTE

Title:	Pseudo-CR on TEID issue in N9 interface

Document for:	Approval

Agenda item:	6.1.5

Spec:	3GPP <TS/TR><TS/TR number>

Version:	<TS/TR version>

Work Item:	<Work Item>

Comments

This contribution proposes KI on TEID issue in N9 interface for SID on security for PLMN hosting a NPN phase 2.

Proposed Changes

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]				3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- [x]				3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

- [y]				Yiming Zhang, et al. “Invade the Walled Garden: Evaluating GTP Security in Cellular Networks”, IEEE Symposium on Security and Privacy (SP), May 2025.

* * * Next Change * * * *

6.X	Key Issue #X: TEID issue in N9 interface

6.X.1	Key issue details

Figure 6.x-1 Example of N9 interface across PLMN operation domain and PNI-NPN operation domain

A PNI-NPN may deploy a UPF, which connects to an PLMN UPF via N9 interface, as shown in Figure 6.x-1. Attackers in PNI-NPN operational domain (e.g., a misbehaving employee in PNI-NPN or an external attacker gaining unauthorized access to the PNI-NPN networks) may obtain the TEID from the UPF deployed in PNI-NPN operational domain.

TS 29.281[x] clause 5.1 states:

Tunnel Endpoint Identifier (TEID): This field unambiguously identifies a tunnel endpoint in the receiving GTP-U protocol entity. The receiving end side of a GTP tunnel locally assigns the TEID value the transmitting side has to use. The TEID value shall be assigned in a non-predictable manner......

TS 29.281[x] just states that TEID shall be assigned in a non-predictable manner, but it does not clearly define how to make TEID unpredictable. As described in [Y],  TEID, a unique identifier of a GTP tunnel, is of 4 bytes. Theoretically, an attacker needs to brute-force a TEID with up to 232 guesses. However, practically, implementation characteristics or flaws often allow for reduced attack space. For example, some implementations may start with an initial TEID value of 1, then increment by one for each new tunnel. Some may not use the full 32-bits for TEID. As a TEID without an established context will trigger error codes in the response while a correct TEID will not, allowing an attacker to guess TEID effectively. If UPF allocate sequential TEID numebers, in this case, an attacker in PNI-NPN operational domain1 can first obtain the TEID assigned by the PLMN UPF to UPF in PNI-NPN operational domain1, then the attack can this information to infer the TEIDs assigned by the PLMN UPF to UPF in PNI-NPN operational domain2 and PLMN gNBs. The attack can further use the TEIDs to  hijack subscriber traffic in other GTP tunnels, as described in the research paper "Invade the Walled Garden: Evaluating GTP Security in Cellular Networks"[Y].

6.X.2	Security threats

Attackers in one PNI-NPN operational domain may forge TEID and infer the TEIDs assigned to PLMN operational domains and other PNI-NPN operational domains  and launch further attacks, such as Dos attack etc.

6.X.3	Potential security requirements

The 5G system shall support a mechanism to assure that TEID is assigned in a non-predictable manner.

The 5G system shall support a mechanism to protect the GTP-U in case of forged TEID.

* * * End of Changes * * * *