3GPP TSG-SA3 Meeting #123																			S3-252653

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2165 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security handling where CU is acting as MN and SN is unchanged |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4_Sec |  |  |  |  |  | Date: |  |  | 2025-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As agreed in RAN WGs, the case where CU is acting as MN and SN is unchanged is in the scope of R19. Currently, the security handling on such case is missing. Security mechanisms and procedures for DC is specified in clause 6.10.2 in TS 33.501. RRC reconfiguration message is used to sync up the SN Counter for KSN derivation. RRC reconfiguration Complete message triggers the security activation in UE and SN. However, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message in LTM case. The SN Counter issue and security activation issue require additional work where CU is acting as MN and SN is unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding clarification on the security handling in the case where CU is acting as MN and SN is unchanged. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.17.2	When DC is configured

In case of inter-SN SCG LTM as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

In the case where CU is acting as MN and SN is unchanged, MN follows the same procedure as specified in the non-DC case. In addition, the MN shall assign a sequence of distinct SN Counter values (maintained for dual connectivity detailed in clause 6.10.3.1 of this document) per candidate gNB to UE during the preparation phase.

During the LTM execution phase, upon receiving the MAC CE message with NCC, the UE shall derive the SN keys with unused SN counter. The UE sends the RRC Reconfiguration Complete to the MN. The UE shall activate the chosen encryption/decryption and integrity protection keys with the SN at this point. In the network side, the MN shall deliver the KSN to SN when KgNB is available. If the KSN is not received in SN and SN received uplink messages, the SN shall defer the messages until the security activation. SN may activate the chosen encryption/decryption and integrity protection with UE on receipt of SN Reconfiguration Complete message or upon receiving the Random-Access request from the UE.

* * * End of Changes * * * *