3GPP TSG-SA3 Meeting #123	S3-252699

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | 0221 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of TC_IE_VALUE_FORMAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE), Montsecure |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test case in chapter 4.3.6.4 does list all requirements for IEs, however does not explicitly define test cases with execution steps and concrete expected results. The status codes are specified to ensure the NF implements the security mechanism needed to fulfil the limitations in 29.501 and the status codes are specified to ensure the NF implements the security mechanisms needed to hold up 29.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case TC_IE_VALUE_FORMAT is split into four test cases for testing the valid format and range of values for IEs in HTTP requests. The execution steps are concretized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The execution of the test case specification by different testers could differ greatly, which could lead to different results and interpretations of the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.3.6.4	The valid format and range of values for IEs

Requirement Name: Validation of the IEs limits.

Requirement Reference: TS 29.501 Principles and Guidelines for Services Definition [13], clause 6.2

Requirement Description:

The valid format and range of values for each IE, when applicable, is defined unambiguously:

-	For each message the number of leaf IEs does not exceed 2048K.

-	The maximum size of the JSON body of any HTTP request does not exceed 16 million octets.

-	The maximum nesting depth of leaves does not exceed 32.

Threat References: TR 33.926 [4], clause 6.3.2.2, JSON Parser not Robust

Test Case:

NOTE 1:	This requirement can also be verified as part of Robustness and Protocol fuzzing tests as defined in clause 4.4.4 Robustness and fuzz testing according to referenced requirements.

Test Name: TC_IE_VALUE_FORMAT

Purpose:

Verify that the API implementation fulfils the requirements as specified in 29.501[13], clause 6.2.

Pre-Conditions:

Test environment with network product under test so that the tester is able to send HTTP requests with "out of bound IEs" towards the network product under test. Rest of the network may be simulated.

NOTE 2:	IEs having invalid format and/or not in the defined range of values can be considered as out of bound IEs.

Execution Steps

Test case 1 (Wrong payload type):

1.	The tester sends an HTTP request with a payload containing an IE value with a type, which is not accepted by the endpoint, to the network product under test. The payload should be valid except for the invalid IE value.

2.	The tester checks if the network product under test responds with an HTTP status code 400 and that it does not contain valid response content for the request.

Test case 2 (JSON object depth exceeds 32):

1.	The tester sends an HTTP request with a JSON object that exceeds 32 levels of nesting to the network product under test.

2.	The tester checks if the network product under test responds with an HTTP status code 422 and that it does not contain valid response content for the request.

Test case 3 (Leaf IEs exceed 2048K):

1.	The tester sends an HTTP request with leaf IEs that exceed 2048K to the network product under test.

2.	The tester checks if the network product under test responds with an HTTP status code 422 and that it does not contain valid response content for the request.

Test case 4 (JSON body size exceeds 16 million octets):

1.	The tester sends an HTTP request with a JSON body that exceeds 16 million octets to the network product under test.

2.	The tester checks if the network product under test responds with an HTTP status code 413 and that it does not contain valid response content for the request.

Expected Results:

Test case 1 (Wrong payload type):

-	The network product under test's response has HTTP status code 400 and its body must not contain valid response content for the request.

Test case 2 (JSON dictionary depth exceeds 32):

-	The network product under test's response has HTTP status code 422 and its body must not contain valid response content for the request.

Test case 3 (Leaf IEs exceed 2048K):

-	The network product under test's response has HTTP status code 422 and its body must not contain valid response content for the request.

Test case 4 (JSON body size exceeds 16 million octets):

-	The network product under test's response has HTTP status code 413 and its body must not contain valid response content for the request.

Expected format of evidence:

-	The used tool(s) name and version information,

-	Settings and configurations used.

-	The output log file of the chosen tool that displays the results (passed/failed).

-	Log/evidence tracing possible crashes.

-	Information of any input causing unspecified, undocumented, or unexpected behaviour.