3GPP TSG-SA3 Meeting #123	S3-252737

Goteborg, Sweden, 25 – 29 August 2025

# Source:	Ericsson

# Title:	New KI - security and privacy aspects of exposure of the standardized data

# Document for:	Approval

Agenda Item:	6.1.7

# 1	Decision/action requested

It is proposed to approve the following changes to [1]

# 2	References

[1]	S3-252564, Skeleton for AIML_CN_Ph2_SEC

# 3	Rationale

This document proposes a new key issue for the AIML_CN_Ph2_SEC TR skeleton [1]. Standardized data over UP for UE data collection in AI/ML-based NR air interface operation may contain sensitive UE information that adversaries could exploit for attacks such as impersonation, traffic interception, and message injection.

Measures to protect the data before exposure to an authorized AF/OTT server outside the 3GPP domain should therefore be considered.

# 4	Detailed proposal

* * * First Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[X]	3GPP TR 23.700-42: "Study on application user consent"

* * * Second Change * * *

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.X	Key Issue #X: Security and privacy aspects of exposure of the standardized data

### 5.X.1	Key issue details

The key issue addresses the security and privacy aspects of exposing standardized data from the UE to the OTT server. Measures to ensure that only authorized and authenticated entities can access such data, and that the data is protected against unauthorized use or modification during exposure need to be considered.

NOTE: User consent aspects for exposure are out of scope of the present document, as application user consent is under study in [X]

### 5.X.2	Security threats

Leakage of sensitive UE-related data to unauthorized entities could lead to impersonation, traffic interception, message injection, modification, or replay attacks.

### 5.X.1	Potential security requirements

The end entities involved in data exposure (e.g., exposure function and OTT server) shall mutually authenticate.

The exposed data shall be confidentiality, integrity and replay protected during the transfer over the exposure interface.

The OTT server shall be authorized before any standardized data is exposed.

* * * End of Changes * * *