3GPP TSG-SA3 Meeting #123	S3-252739

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2169 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling mismatch of security capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4_Sec |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It’s not clear which condition triggers updating UE with LTM configuration in network. (if it’s the case that intra-gNB-CU handover is not initiated or the target gNB releases LTM configuration) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added specific condition if it’s the case that intra-gNB-CU handover is not initiated or the target gNB releases LTM configuration. - In the last sentence, the term ‘serving gNB’ can be aligned to ‘the target gNB’ considering context of the paragraph. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It brings misunderstanding if not corrected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** CHANGE starts ***

6.17	Security mechanism and procedures for L1/L2 Triggered Mobility

6.17.1	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], during LTM preparation phase, the serving gNB shall send the {KNG-RAN*, NCC} pair (per candidate cell), UE's 5G security capabilities, ciphering and integrity algorithms used in the serving cell and the UE's UP security policy along with the UE security context to the candidate gNB(s). The candidate gNB(s) shall send the LTM configuration(s) (containing the selected AS security algorithms and the UE’s UP security activation status) to the serving gNB for the accepted candidate cell(s). The serving gNB shall send the RRCReconfiguration message to the UE including the LTM candidate configurations containing the selected AS security algorithms and the UE’s UP security activation status.

The {KNG-RAN*, NCC} pair at the candidate cells require updating when a new key at the serving gNB, e.g. due to an inter-CU handover/cell switch or an intra-CU handover/cell switch with a change of key. To update the candidate cells, the serving gNB shall send the generated {KNG-RAN*, NCC} pair (per candidate cell) to the candidate gNB(s) using LTM Configuration Update message.

During LTM execution phase, the serving gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE. Upon receiving the cell switch command, the UE derives the KNG-RAN* as described in clause 6.9.2.3.4 of the present document and switches to the target gNB.

NOTE: Key changes that use the keySetChangeIndicator field to be set to true use the normal L3 handover.

If the target gNB receives UE's 5G security capabilities from the AMF (as detailed in clause 6.7.3.1 of the present document) and/or UE's UP security policy from the SMF (as detailed in clause 6.6.1 of the present document) in the Path-Switch Acknowledge message, then the target gNB may initiate an intra-gNB-CU handover to refresh KNG-RAN* or activate or de-activate the UP integrity/confidentiality as per the received policy from SMF or to indicate the selected algorithms appropriately. Further, the target gNB either releases or updates the LTM configuration (UE's 5G security capabilities and/or UE's UP security policy) of the candidate cells. (When intra-gNB-CU handover is not initiated) or (when the target gNB releases LTM configuration), the new serving gNB (the target gNB during LTM execution) then updates the UE with the configuration aligned with LTM configuration in the network.

*** CHANGE ends ***