3GPP TSG-SA3 Meeting #123																			S3-252744

Goteborg, Sweden, 25 – 29 August 2025

# Source:	OPPO

# Title:	New Key issue on security protection in split MME architecture

# Document for:	approval

Agenda Item:	6.1.10

1	Decision/action requested

It is proposed to approve New Key issue on security protection in split MME architecture.

2	References

N/A

3	Rationale

It is proposed to approve the new key issue on security protection in split MME architecture.

4	Detailed proposal

* * * First Change * * * *

5.X	Key Issue #X: Security protection in split MME architecture

5.X.1	Key issue details

According to S3-252359 [x], the satellite communication industry had already proposed the requirement for UE to be able to contact multiple satellites in each S&F operation round, which means an authenticated UE can exchange NAS messages with multiple satellites without performing UE authentication or key negotiation (NAS SMC) between the UE and satellites.

Allowing a UE to interact with a set of MME-onboard(s) means the UE security context is synchronized across the MME-onboard(s), which is the current assumption under Rel-19 TS 23.401 [y]. However, the only exception to the synchronization of NAS security context is the NAS COUNT, which may or may not be synchronized between the MME-onboards. In either case, there are security risks. For example, in case of synchronized NAS COUNT, it may lead to the security risk of key stream reuse; in case of not synchronized NAS COUNT, it may lead to the replay attacks.

In this key issue it is proposed to study potential solutions to prevent the S&F operation in split MME architecture from the security risks of the key stream reuse and replay attacks.

5.X.2	Security threats

Based on whether the NAS COUNT is synchronized across the MME-onboards, the following threats can manifest themselves:

-    If the NAS COUNT is synchronized across the MME-onboard(s), i.e., there is only one pair of NAS COUNT, it may lead to the security risk of key stream reuse.

-    If the NAS COUNT is not synchronized across the MME-onboard(s), i.e., there are separate pairs of NAS COUNTs per MME-onboard, it may lead to the replay attacks.

5.X.3	Potential security requirements

The 3GPP system shall support means to prevent the key stream reuse among MME-onboard(s).

The 3GPP system shall support means to prevent the replay attacks of NAS messages between UE and MME-onboard(s).

* * * Second Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	S3-252359, New SID on Security Aspects of 5G Satellite Access Phase 4.

[y]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

* * * End of Changes * * * *