3GPP TSG-SA3 Meeting #123	S3-252755

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Samsung

Title:	Resolving EN on KAIOTF key

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

When deriving KAIOTF, RANDAIOT_d that the AIoT device generated is used for freshness.

* * * First Change * * * *

## 5.3	Protection of information during AIoT service communication

### 5.3.1	General

This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection.

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection, using RANDAIOT_d.

Editor’s Note: How to prevent key stream reuse is FFS.

3.	The AIoTF shall construct a NAS Command Request and protect the message based on the KAIoTF, the confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the Command Request containing the protected NAS Command Request to NG-RAN.

Editor’s Note: the selection of confidentiality and integrity algorithms by the network is FFS

4.	The NG-RAN shall send a R2D message containing the protected NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6] .

5.	The device shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KAIoTF key using the same algorithms.

6.	The AIoT device shall send a D2R message containing the protected NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the Command Response containing the protected NAS Command Response to the AIoTF.

8-9.	The AIoTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

* * * End of Changes * * * *