3GPP TSG-SA3 Meeting #123	S3-252763

General

Secretary Remarks

History

# Goteborg, Sweden, 25 – 29 August 2025

Source:	China Mobile

Title:	Impact of PQC algorithm transitioning on terminals

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.0.0

Work Item:	FS_CryptoPQC

Comments

This contribution presents principles of PQC transitioning on terminals, and analysis the suitability of lattice-based and hash-based algorithms for terminals.

* * * First Change * * * *

<Proposed change in revision marks>

5		Principles and attributes of PQC to use in 3GPP procedures

The security protocols of communication systems should allow terminals to adopt either hybrid or standalone PQC modes, in order to accommodate the capabilities of networks and terminals as well as policy requirements.

Lattice-based algorithms offer smaller signature and public key sizes, as well as higher computational efficiency, making them suitable for mobile terminals and IoT devices.

Hash-based algorithms have a well-established theoretical foundation, and provide high security. But their signature size is larger and signing speed is slower, making them suitable for terminals which are not performance-sensitive but require long-term security protection.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *