3GPP TSG-SA3 Meeting #123	S3-252768

# Goteborg, Sweden, 25 – 29 August 2025

# Source:	Huawei, HiSilicon

# Title:	Discussion on security of UE-Satellite-UE in IMS

# Document for:	Discussion

Agenda Item:	4.1.2

1	Decision/action requested

It is proposed to discuss the security threat of UE-Satellite-UE in IMS.

2	References

[1]	TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[2]	TS 33.328: "IP Multimedia Subsystem (IMS) media plane security (Release 18)"

3	Rationale

3.1	Background of UE-Satellite-UE communication in IMS

3.1.1 Deployment of IMS-AGW

Figure 1 shows the architecture of UE-Satellite-UE communication in IMS with IMS-AGW deployed on satellite, specified in Clause AE.2.1 of TS 23.228 [1]. In order to support the UE-Satellite-UE communication, the IMS-AGW that is used to route the media plane data should be deployed on the satellite(s) that host the gNB and UPF (UL CL/BP and L-PSA) of the 5GC.

Figure 1: Reference architecture of IMS satellite media plane optimization

3.1.2 Relocation of IMS-AGW

Figure 2 shows the satellite handover procedure in UE-Satellite-UE communication in IMS, which is specified in AE.5.2.1 of TS 23.228. At step1, the IMS media data is routed via the IMS AGW on the source satellite. After the satellite that serves the UE changes from the source satellite to the target satellite procedure (i.e., steps 2~17), the IMS media plane data is routed via the IMS AGW on the target satellite (i.e., step 18). It implies that when the satellite changes from the source satellite to the target satellite, the IMS-AGW is relocated accordingly.

Figure 2: media routing path change due to change of satellites

3.2	Background of the security association set-up procedure in IMS

According to clause 7.2.1 of TS 33.328, when the protection of RTP using SDES is chosen, the UE will send the SDES crypto attributes containing the master key K11 to P-CSCF via the SDP offer message (see Figure 3). After that, the P-CSCF will send the SDES crypto attributes containing master key K11 to the IMS-AGW. Based on the master key K11, the IMS AGW is able to to check integrity / decrypt the media plane mesasge arriving from IMS UE A. Note that TS 33.328 does not define any IMS-AGW handover procedure, because the IMS-AGW is considered not to be changed during the transmission of media plane mesasge.

Figure 3: security association set-up procedure for e2ae case

3.3	Observation

At the beginning of the Satellite study, the UE-Satellite-UE communication in IMS is thought to be secure by just reusing e2ae protection defined in TS 33.328 [2]. However, as described in the above clause 3.2, the e2ae protection only applies to the scenario when the IMS-AGW does not change, while according to clause 3.1.2, the IMS-AGW in UE-Satelite-UE scenario changes frequently. This implies that if e2ae protection of RTP using SDES is simply reused, the same master key from IMS UE A will be distributed to the IMS-AGW on different satellites. It fails to follow the principle that the keys should be isolated among different entities.

Observation: For e2ae protection of RTP using SDES, Master Keys between the UE and the IMS-AGW on different satellites are the same, violating the rule that keys should be isolated among different entities.

4	Detailed proposal

It is proposed to discuss the requirement that the keys between the UE and the IMS-AGW on the satellites should be isolated, and revise the SID of satellite to protect the UE-Satellite-UE communication in R20.