3GPP TSG-SA3 Meeting #123	S3-252769

Goteborg, Sweden, 25 – 29 August 2025

Source:	CATT

Title:	Pseudo-CR on New Key Issue for PSK Delivery for MPQUIC in ATSSS

Document for:	Endorsement

Agenda item:	6.1.6

Comments

MPQUIC in ATSSS (as defined in TS 23.501 [1], clause 5.32.6.2.2) uses QUIC over TLS. However, the existing specification in Annex AA of TS 33.501 [2] limits UE and UPF authentication to X.509 certificates.

In this new study item on security aspects for QUIC/TLS [3], the work task is as follows:

WT#1: Study the support of PSK mode, in particular:

- Study key derivation and delivery for UPF.

NOTE: the impact to the 5GS should be minimized.

To use the QUIC 0-RTT feature, a pre-shared key (PSK) is required. PSK is the basis for Early Data (0-RTT), which is an authentication method in TLS 1.3 where client and server share a secret key in advance.

This key issue addresses the lack of standardized procedures for delivering PSKs for use in MPQUIC/TLS for ATSSS.

Proposed Changes

* * * First Change * * * *

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 3GPP TS 23.501: "System Architecture for the 5G System".

[3] 3GPP TS 33.501: "Security architecture and procedures for 5G System".

[4] S3-252354: "New SID on security aspects for QUIC/TLS".

[5] IETF RFC 9001: "Using TLS to Secure QUIC".

* * * Next Change * * * *

# 5	Key issues

## 5.X	Key Issue #X: PSK Delivery for MPQUIC in ATSSS

### 5.X.1	Key issue details

MPQUIC in ATSSS (TS 23.501 [2], clause 5.32.6.2.2) uses QUIC secured by TLS. For PSK mode to be usable, the UPF and the UE need to obtain the PSK in time for QUIC/TLS setup for MPQUIC in ATSSS. The existing specifications do not define a PSK delivery mechanism or how PSK delivery does not conflict with existing specifications. This key issue addresses the lack of standardized procedures for delivering PSKs for use in MPQUIC for ATSSS.

The study should address at least:

- How to securely deliver the pre-shared key to the UPF and to the UE.

- How to ensure PSK delivery procedures not conflicting with the existing specification

### 5.X.2	Security threats

Not applicable.

### 5.X.3	Potential security requirements

Not applicable.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *