3GPP TSG-SA3 Meeting #123	S3-252771

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.513 | CR | Draft CR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 33.513 based on GSMA NESASG agreements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to GSMA NESASG agreements, several corrections to 33.513 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Complementing abbreviations not listed in TR 21.905. - Remove H-UPF from the simulated functions. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity for readers. Low quality of SCAS documents. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.2.1, 4.2.2.3, 4.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** START of CHANGES****************

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ESP	Encapsulating Security Payload

gNB	gNodeB

H-SMF	Home-Session Management Function

H-UPF	Home-User Plane Function

IKE	Internet Key Exchange

IPUPS	Inter-PLMN User Plane Security

SBA	Service Based Architecture

SBI	Service Based Interfaces

SEG	Security Gateway

SMF	Session Management Function

V-SMF	Visited-Session Management Function

**************** END of 1st CHANGE****************

**************** START of 2nd CHANGE****************

4.2.1	Introduction

The security functional requirements and the related test cases specific for UPF are described in the following clause.

**************** END of 2nd CHANGE****************

**************** START of 3rd CHANGE****************

4.2.2.3	Replay protection of user data transported over N3 interface

Requirement Name: Replay protection of user data transported over N3 interface

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: The transported user data between gNB and UPF is replay protected as specified in TS 33.501, clause 9.3.

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data"

TEST CASE:

NOTE 1:	This test case is only applicable to UPF supporting IPSec in N3 interface without the use of a SEG.

Test Name: TC_UP_DATA_REPLAY_UPF

Purpose:

Verify that the transported user data between gNB and UPF are replay protected.

Procedure and execution steps:

Pre-Condition:

-	UPF network product is connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the original user data transported via N3 reference point between gNB and UPF.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported between UE and UPF is replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

**************** END of 3rd CHANGE****************

**************** START of 4th CHANGE****************

4.2.2.7	IPUPS

Requirement Name: IPUPS packeting handling

Requirement Reference: TS 33.501[2], clause 5.9.3.4

Requirement Description:

The IPUPS only forwards GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others as specified in TS 33.501 [2], clause 5.9.3.4.

Threat Reference: TR 33.926 [7], Clause L.2.5, "invalid user plane data forwarding"

TEST CASE:

NOTE 1:	This test case is only applicable to UPF supporting IPUPS.

Test Name: TC_IPUPS_PACKET_HANDLING

Purpose:

Verify that the packets not belonging to an active PDU session are discarded.

Pre-Conditions:

Test environment is set up with a V-SMF, an H-SMF, and a gNB which may be simulated.

Execution Steps:

1)	The V-SMF requests the UPF with IPUPS functionality under test to establish an N4 session for a PDU session in home-routing roaming. The UPF with IPUPS functionality under test responds to the SMF with the F-TEID for the N9 tunnel towards the H-UPF, and the F-TEID for the N3 tunnel towards the gNB.

2)	The V-SMF requests the H-SMF to establish a PDU session providing the received F-TEID for the N9 tunnel.

3)	The H-SMF requests the H-UPF to establish an N4 session providing the received F-TEID for the N9 tunnel. H-UPF in the response provides its F-TEID for the N9 tunnel. The H-SMF provides the received F-TEID from the H-UPF to the V-SMF.

4)	The V-SMF requests the gNB to allocate resource for the PDU session providing the F-TEID for the N3 tunnel received at step 1. The gNB replies with its F-TEID for the N3 tunnel to the V-SMF.

5)	The V-SMF provides the UPF with IPUPS functionality under test with the received F-TEID assigned by the gNB for the N3 tunnel and the received F-TEID assigned by the H-UPF for the N9 tunnel.

6)	 The H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel.

7)	The H-UPF is triggered to send GTP-U packets using an F-TEID different than the one assigned by V-UPF for N9 tunnel.

Expected Results:

When the H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel (step 6 in the execution steps), GTP-U packets are witnessed over the N3 tunnel.

When the H-UPF is triggered to send GTP-U packets using an F-TEID different than the one assigned by the V-UPF (step 7 in the execution steps), no GTP-U packets are witnessed over the N3 tunnel.

Expected format of evidence:

Files recording the GTP packets captured (e.g. pcap trace).

**************** END of CHANGES****************