3GPP TSG-SA3 Meeting #123	S3-252791

# Goteborg, Sweden, 25 – 29 August 2025

Source:	China Mobile

Title:	Pseudo-CR on Usage mode of quantum-safe cryptographic algorithms in 6G systems

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.0.0

Work Item:	FS_CryptoPQC

Comments

- Introduction

This contribution proposes a sub-clause of the usage mode of quantum-safe cryptographic algorithms in 6G systems .

- Reason for Change

A CRQC(Cryptographically Relevant Quantum Computer) raises threats to conventional cryptograhpic algorithms. Quantum-safe cryptographic algorithms have to be deployed in 6G systems to resist against quantum computing attacks. The usage mode of quantum-safe cryptographic algorithms should be determined at the beginning of the design of 6G systems.

* * * First Change * * * *

5		Principles and attributes of PQC to use in 3GPP procedures

5.X	Usage mode of quantum-safe cryptographic algorithms in 6G

A CRQC(Cryptographically Relevant Quantum Computer) is a large-scale, fault-tolerant quantum computer with the ability to crack the asymmetric cryptographic schemes (e.g., RSA or ECC) by running Shor algorithm, or  to halve security strength of  symmetric cryptographic schemes (e.g., AES) by running  Grover algorithm.The 2023 Quantum Threat Timeline Report [X] comprehensively analyzes the timeline of how quantum computing may disrupt current cryptographic systems. The report suggests that the likelihood of CRQC occurring by 2030 exceeds 50%.

Symmetric cryptographic algorithms with 256-bit key length are usually considered to be quantum-safe, such as AES-256, SNOW 5G, and ZUC-256. Currently most used asymmetric cryptographic schemes (e.g., RSA or ECC) have to be abandoned when CRQC becomes true, as they can be broken by using CRQC. For this, Since 2016 NIST has made many efforts to promote the standardization of post quantum algorithm (PQC),i.e. quantum-safe asymmetric cryptographic algorithms.

6G system is the next generation mobile communication system which is anticipated to be released in 2030. It is usually composed of 6G UE and 6G network. Quantum-safe cryptographic algorithms, including 256-bit symmetric cryptographic algorithms and PQC, can be applied to 6G system in two modes: standalone mode and hybrid mode. The former means that only quantum-safe cryptographic algorithms are utilized, while the latter means that not only quantum-safe cryptographic algorithms but also conventional  cryptographic algorithms are applied.

There are following four cases to the usage mode of quantum-safe cryptographic algorithms in 6G system:

- － Case 1: 6G UE applies standalone mode, and 6G network applies standalone mode;

- －  Case 2: 6G UE applies standalone mode, and 6G network applies hybrid mode;

- －  Case 3: 6G UE applies hybrid mode, and 6G network applies hybrid mode;

- －  Case 4: 6G UE applies hybrid mode, and 6G network applies standalone mode.

Which case is well suited to 6G system has to be identified at the initial phase of the standard work at 6G security. The following factors may affect the final decision:

- Conventional systems have to migrate to be quantum-safe before 2035;

- 6G system is completely new that will be commercially available  in 2030;

- 6G network does not serve 5G UE;

- 5G UE does not connect 6G network.

* * * Next Change * * * *

2	References

[X]	 Quantum Threat Timeline Report, Global risk institute,2023

https://globalriskinstitute.org/mp-files/quantum-threat-timeline-report-2023.pdf/

* * * End of Changes * * * *