3GPP TSG-SA3 Meeting #123																			S3-252806

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Addressing Editor’s Notes in the authentication procedure

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IOT_Sec

Comments

This contribution proposes to update the authentication procedure. The updated procedure describes how the AIoT device authentication by the network is performed using the inventory procedure with AIoT device identifier. The authentication procedure also considered AIoT device identifier privacy since the procedure is based on the inventory procedure. For mutual authentication, how the AIoT device authenticates the network is described in S3-252812.

The proposed updates address the following editor’s notes.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

Editor’s note: How and where to derive keys is FFS.

The above Editor’s Notes are deleted as the updated procedure proposes to have AIOTF hold the KAIoTF and generate a RANDAIOT_n  and temporary ID (i.e., T-ID). The proposal is intended to reduce the signaling overhead between AIOTF and UDM. We note that if AIOTF does not have a valid KAIoT, the bootstrapping procedure is performed as proposed in S3-252807.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

The above Editor’s Note is deleted as we described how RESAIOT is computed in the procedure.

Editor’s Note: Whether replay attack is possible is FFS.

Editor’s Note: The security requirements of generating RANDAIOT_d are FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

The above Editor’s Notes are deleted as for authentication of a signle AIoT device, the AIoT deevice uses self-generated RANDAIOT_d in calculation of RESAIOT and in such case the replay attack is not valid.

Editor’s Note: Where the authentication credentials are processed in AIOT device is FFS.

The above Editor’s Note is deleted based on the SA’s guidance in SP-250852, where the processing of authentication credential in AIoT device is considered outside the scope of 3GPP specifications.

Lastly, we defined the function FA for temporary ID derivation and RESAIoT calculation. Defining the cryptographic algorithm to realize FA (e.g., HMAC-SHA-256) is not the purpose of this contribution and can be discussed separately.

Proposed Changes

# * * * * First change * * * *

## 5.2	Authentication procedure

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network.

Editor’s Note: The alignment with the ID privacy procedure is FFS.

NOTE: KAIOT is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0.	Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed. The received AIoT Identification Information includes an AIoT device identifier(s).

1.	The AIOTF shall retrieve the KAIoTF for the AIoT device ID.

If the KAIoTF is not available locally, the AIOTF shall perform a key bootstrapping procedure to establish the KAIoTF.

The AIOTF shall generate a random nonce, RANDAIOT_n and compute a temporary ID (T-ID) using the KAIoTF, the AIoT device ID and RANDAIOT_n as specified in Annex A.xy.

2.	The AIOTF shall send Inventory Request message including the T-ID, RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3.	The NG-RAN shall send the Paging message including T-ID and RANDAIOT_n to the AIoT device.

4.	Upon receiving the Paging message, AIoT device shall compute a T-ID as the AIOTF did in step 2 and check if the computed T-ID matches with the received T-ID. If it matches with the received one, the AIoT device shall derive RESAIOT using KAIoTF, RANDAIOT_n and a randomly generated nonce RANDAIOT_d as specified in Annex A.xz for network authenticating AIoT Device.

5.	The AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d.

6.	NG-RAN shall the Inventory report message to the AIOTF, including the RESAIOT and RANDAIOT_d.

7.	 Upon receiving the Inventory Report, the AIOTF shall compute a XRESAIOT in the same way RESAIOT was computed in the AIoT device, as specified in Annex A.xz. The AIOTF compares the received RESAIOT with XRESAIOT.

8.	If the XRESAIOT matches with the received RESAIOT, the AIOTF shall continue the steps 12-14 in clause 6.2.2 for inventory procedure or the steps 8-11 of clause 6.2.3 for command procedure in TS 23.369 [2].

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

# * * * * Second change * * * *

# A.xx	Function FA

The function FA is defined as follows:

Output = FA(key, input parameters)

Editor’s Note: the details of function FA is FFS.

# * * * * Third change * * * *

# A.xy	T-ID generation

When generating the T-ID, the following parameters are used as input to the FA as defined in Annex A.xx:

-	AIoT device ID;

-	RANDAIOT_n

If the T-ID generation is used for key bootstrapping as specified in clause 5.A, the input key shall be KAIoT. Otherwise, the input key shall be KAIoTF.

# * * * * Fourth change * * * *

# A.xz	RESAIoT calculation

When calculating RESAIoT, the following parameters are used as input to the FA as defined in Annex A.xx:

-	AIoT device ID;

-	RANDAIOT_n;

-	RANDAIOT_d

The input key shall be KAIoTF.

# * * * * End of change * * * *