3GPP TSG-SA3 Meeting #123																			S3-252808

Goteborg, Sweden, 25 – 29 August 2025

Source:	Qualcomm Incorporated

Title:	Privacy protection of device ID in individual inventory

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	Ambient_IoT_Sec

Comments

This contribution proposes an updated individual inventory procedure (i.e., inventory with AIoT device identifier) that protects AIoT device identifier privacy.

Note that the proposed procedure is identical to the authentication procedure described in S3-252806  as the authentication procedure is based on the Inventory procedure and already considered the identity privacy in sending the Inventory request and constructing the RESAIOT.

Also note that the proposed ID privacy mechanism is based on the option A in the living document (S3-252326) with updates to address the editor’s notes.

The proposed updates address the following Editor’s Notes in the option A of the living document:

Editor’s Note: whether AIoTF or ADM computes T-ID is FFS.

Editor’s Note: whether AIoTF or ADM computes T-ID’ is FFS.

Editor’s Note: in case AIoTF computes T-ID, a key KAIoTF derived from KAIoT in ADM is used. How AIOTF retrieves the KAIoTF is FFS.

Editor’s Note: in case the T-ID is computed by the ADM, whether the cryptographic key is the long-term key KAIoT or a key derived from KAIoT , and the impact of interaction between AIOTF and ADM and the analysis of load of ADM is FFS.

Editor’s Note: which input key (e.g., KAIoT or KAIoTF) to be used is FFS

The above Editor’s Notes are deleted as the updated procedure proposes to have AIOTF hold the KAIoTF and generate a temporary ID (i.e., T-ID) and RESAIOT that corresponds to T-ID’ in the Editor’s Note. The proposal is intended to reduce the signaling overhead between AIOTF and UDM. We note that if AIOTF does not have a valid KAIoT, the bootstrapping procedure is performed as proposed in S3-252807.

Editor’s Note: whether and how to address attacks of an attacker broadcasting T-ID and Nonce triggering all AIoT Devices to constantly compute T’-D (e.g., energy depletion in the AIoT devices) is FFS.

If an attacker constantly triggers Paging message with random T-ID and Nonce, the nearby AIoT devices would compute a T-ID and check if it is matched with the received T-ID for each Paging message. However, energy consumption of T-ID computation is not an issue because AIoT devices are assumed to harvest enough energy from RF signals for backscattering transmission. As long as the AIoT device can receive and decode Paging messages, the AIoT device should be able to perform T-ID computation. Therefore, it is proposed to remove the above Editor’s Note.

Editor’s Note: how to address the attack that manipulates the RANDAIOT_N in the Paging message is FFS.

If an attacker manipulates the RANDAIOT_N in the Paging message, the computed T-ID would not be matched with the received T-ID. Consequently, the AIoT device stops further processing of paging message as described in step 2. Therefore, it is impossible for an attacker to receive a valid Paging response from AIoT devices if the attacker manipulated the RANDAIOT_N. In addition, the Paging message manipulated by the attacker does not affect subsequent Inventory procedure as AIoT devices do not maintain the received RANDAIOT_N. Therefore, it is proposed to remove the above Editor’s Note.

Lastly, we defined the function FA for temporary ID derivation and RESAIoT calculation. Defining the cryptographic algorithm to realize FA (e.g., HMAC-SHA-256) is not the purpose of this contribution and can be discussed separately.

Proposed Changes

# * * * * First change * * * *

## 5.4	Protection of AIoT device identifier privacy

### 5.4.1	General

This clause describes the mechanism to protect AIoT device identifier privacy during the inventory procedure based on a temporary ID. The temporary ID is generated based on the key (i.e., KAIoTF) shared between AIoT device and AIOTF.

### 5.4.2	Procedure for AIoT Device identifier protection for inventory with AIoT device ID

Figure 5.4.2-1:  Privacy protection of AIoT device ID in inventory with AIoT device ID

0.	Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed. The received AIoT Identification Information includes an AIoT device identifier(s).

1.	The AIOTF shall retrieve the KAIoTF for the AIoT device ID.

If the KAIoTF is not available locally, the AIOTF shall perform a key bootstrapping procedure to establish the KAIoTF.

The AIOTF shall generate a random nonce, RANDAIOT_n and compute a temporary ID (T-ID) using the KAIoTF, the AIoT device ID and RANDAIOT_n as specified in Annex A.xy.

2.	The AIOTF shall send Inventory Request message including the T-ID, RANDAIOT_n to NG-RAN.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3.	The NG-RAN shall send the Paging message including T-ID and RANDAIOT_n to the AIoT device.

4.	Upon receiving the Paging message, AIoT device shall compute a T-ID as the AIOTF did in step 2 and check if the computed T-ID matches with the received T-ID. If it matches with the received one, the AIoT device shall derive RESAIOT using KAIoTF, RANDAIOT_n and a randomly generated nonce RANDAIOT_d as specified in Annex A.xz for network authenticating AIoT Device.

5.	The AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d.

6.	NG-RAN shall the Inventory report message to the AIOTF, including the RESAIOT and RANDAIOT_d.

7.	Upon receiving the Inventory Report, the AIOTF shall compute a XRESAIOT in the same way RESAIOT was computed in the AIoT device, as specified in Annex A.xz. The AIOTF compares the received RESAIOT with XRESAIOT.

8.	If the XRESAIOT matches with the received RESAIOT, the AIOTF shall continue the steps 12-14 in clause 6.2.2 for inventory procedure or the steps 8-11 of clause 6.2.3 for command procedure in TS 23.369 [2].

# * * * * Second change * * * *

# A.xx	Function FA

The function FA is defined as follows:

Output = FA(key, input parameters)

Editor’s Note: the details of function FA is FFS.

# * * * * Third change * * * *

# A.xy	T-ID generation

When generating the T-ID, the following parameters are used as input to the FA as defined in Annex A.xx:

-	AIoT device ID;

-	RANDAIOT_n

If the T-ID generation is used for key bootstrapping as specified in clause 5.A, the input key shall be KAIoT. Otherwise, the input key shall be KAIoTF.

# * * * * Fourth change * * * *

# A.xz	RESAIoT calculation

When calculating RESAIoT, the following parameters are used as input to the FA as defined in Annex A.xx:

-	AIoT device ID;

-	RANDAIOT_n;

-	RANDAIOT_d

The input key shall be KAIoTF.

# * * * * End of change * * * *